Theory Manual

for the

MYSTRAN General Purpose

Finite Element Structural Analysis

Computer Program

(Open Source Version)

Dr. Bill Case – Original Author

Current Contributors (in alphabetical order):

Steve Doyle, Brian Esp, Zach Lerner, Bruno Paschoalinoto

www.mystran.com

(April 2024)

Most consistent with MYSTRAN program version X

Contents

1	MYSTRAN Sample Problem	2

2	Equations for the reduction of the G-set to the A-set and solution for displacements and constraint forces	19

2.1	Introduction	20

2.2	Reduction of the G-set to the N-set	20

2.3	Reduction of the N-set to the F-set	22

2.4	Reduction of the F-set to the A-set	23

2.5	Reduction of the A-set to the L-set	25

2.6	Solution for constraint forces	25

3	 Equations for Element Stress/Strain Recovery	29

3.1	General discussion	30

3.2	Rod element	30

3.3	Bar element	31

3.4	Plate elements	33

3.4.1	Membrane stresses	33

3.4.2	Bending stresses	34

3.4.3	Combined membrane and bending stresses	34

3.4.4	Transverse shear stresses	34

3.5	Solid elements	35

4	Craig-Bampton Model Generation	36

4.1	Craig-Bampton Equations of Motion for Substructures	37

4.2	Development of Displ  Output Transformation Matrices (Displ OTM’s)	42

4.3	Development of Load Output Transformation Matrices (Load OTM’s)	45

4.3.1	LTM Terms for Substructure Interface Forces	45

4.3.2	LTM Terms for Net cg Loads	45

4.3.3	LTM Terms for Element Forces and Stresses	47

4.3.4	LTM Terms for Grid Point Forces due to multi-point constraints (MPC’s)	47

4.4	Development of Acceleration Output Transfer Matrices (Accel OTM)	50

4.5	Correspondence between matrix names and CB Equation Variables	51

4.6	Craig-Bampton model generation example problem	53

4.6.1	CB-EXAMPLE-12-b.F06	56

4.6.2	OUTPUT4 matrices written to CB-EXAMPLE-12-b.OP1 and OP2	65

4.6.3	Displ and Element force/stress OTM’s written to CB-EXAMPLE-12-b.OP8 and OP9	68

5	 Derivation of RBE3 Element Constraint Equations	75

5.1	Introduction	76

5.2	Equations for translational force components	78

5.3	Equations for rotational moment components	80

5.4	Summary of equations for the RBE3	85

6	Equations for the BUSH Element	86

# MYSTRAN Sample Problem

This example problem shows the input and output for a simple rod with 7 grids and 6 elements.  The rod is subjected to loads in two subcases as described below:

Z0, X13

6

5

4

3

2

1

701

601

501

401

301

201

101

Yo, Z13

The basic coordinate system is the X0, Y0, Z0 system shown (with X0 in the direction of Yo cross Z0).  In addition, rectangular coordinate system X13, Y13, Z13 (with X13 in the same direction as Z0)  is also shown and will be used in the input data in order to help explain the use of coordinate systems.  The basic system does not have to be defined explicitly.  It is implied through the model grid coordinates and any other coordinate systems (other than basic) which might be referenced in field 3 of the Bulk Data GRID entry.  Coordinate system 13 must be defined via a CORD2R Bulk data entry.

The grid point IDs are 101-701 and the rod element IDs are 1-6.  The total length is 60 inches consisting of 6 elements of 10 inches each.  All of the rods have the same cross-sectional area of 0.6 inch2.  The material is aluminum with a Young’s modulus of 1x107.  The model is constrained at the left end.  Several loads are applied in two subcases.

Subcase 35  consists of a 120 lb load at grid 701

Subcase 8  consists of a 240 lb load at grid 201, a 150. Lb load at grid 301 and a 200 lb load at grid 401

The output, which includes an echo of the input data deck, is shown on the following pages.  Note the following about the OUTPUT:

- The input data consists of everything from the ID entry through the ENDDATA entry, and consists of the Executive Control, Case Control and Bulk Data Decks.  Entries that begin with a $ sign (and have anything after $ in the entry) are commentary and are ignored.

- The Executive Control Deck begins with the optional ID entry, has the mandatory SOL entry (1 for statics) and ends with the mandatory CEND entry.  All Executive Control entries are free field in that they may be anywhere within the 80 columns of an entry.

- The Case Control Deck begins with the entry following CEND (which in this case is a TITLE Case Control entry) and ends with the mandatory BEGIN BULK entry.  The entries in between can be in any order that makes sense.  That is, if there are no subcases, the data can be in any order.  When there are subcases, as is the case for this example, the entries between one SUBCASE entry and another apply only to that subcase.  Anything “above” the subcase level pertains to all subcases, unless overridden in a subcase.  All Case Control entries are free field.

- The SPC = 19 entry requests that a Bulk Data SPC (or SPC1, SPCADD) with set ID = 19 be used in defining the single point constraints for the model.

- The following three entries request various outputs (displacements, etc) with ALL meaning that displacements for all grids (DISP = ALL), applied loads for all grids (OLOAD = ALL) and forces of single point constraint (SPCF = ALL).  As these are “above” the subcase ;evel, they apply to all subcases (unless a subcase requests output of the same type for a different set of grids or elements)

- Subcase 35 (the first subcase in Case Control) is defined with its own subtitle and with LOAD = 191 requesting that a Bulk Data entry with set ID of 191 define the loads for this subcase (which requires that the load be defined on a LOAD, FORCE, MOMENT, GRAV od PLOAD2 Bulk Data entry).  In this case, Bulk Data entry FORCE with a set ID od 191 contains the load definition for this subcase.  Element engineering force and stress output is requested for this subcase (in addition to the requests above the subcase level). .

- Subcase 8 (the second subcase in Case Control) is defined with its own subtitle (notice the order doesn’t matter) and requesting load set 26 in Bulk Data to define the load.  There is also another output request (for nodal element forces) for set 98.  Set 98 is defined as  2,5.  Since set 98 output is requested as element forces, the 2,5 is interpreted as the element numbers for which nodal element forces will be output in this subcase only.  If the request had been above the subcase level (as DISP = ALL, etc) the request would have been honored for both subcases.

- The Bulk Data Deck begins with the entry immediately following BEGIN BULK and ends with the mandatory ENDDATA entry.  The logical entries in between can be in any order with the exception that any one logical entry must be in order.  Thus the MAT1 logical entry, which has one parent entry and one continuation entry must be entered together and in the order shown.

- Coordinate system 13 is defined on the CORD2R Bulk Data entry with 13 as the coordinate system ID in field 2.  The reference system in field 3 is, in this case, the basic system.  It does not have to be.  Coordinate system 13 could use some other coordinate system as its reference, and so on.  However, the last system in the chain would have to have the basic system as its reference.  The nine real numbers on the remainder of the CORD2R logical entry describe three points in coordinates of the reference (basic) system.  The first three numbers are the coordinates of the origin of coordinate system 13, which is at the origin of the basic system.  The next three numbers are the coordinates of a point on the Z13 axis, which is in the direction of the Y0.  The next three numbers (on the continuation entry) are the coordinates of a point in the X13 – Z13 plane.  Thus it is seen that this CORD2R entry describes coordinate system 13 as seen on the figure above.

- The seven grid points of the model are defined on the GRID entries.  Note that field 3 (coordinate systems for grid coordinates) is blank indicating the basic coordinate system for grid locations for all seven grids.  Field 7, the global coordinate system for each grid is also the basic system for grids 101 through 601.  Grid 701, however uses coordinate system 13 as its global system.  Field 8 of the GRID entries is for “permanent” single point constraints.  Note that 13456 are the permanent single point constraints for grids 101 - 601.  Since the rod can only take axial load and torque, only global degrees of freedom that are for displacement along the rod, or rotation about its axis can possibly have stiffness.  Since grids 101 - 601, have the basic system as global, degrees of freedom 1346 will be singular and must therefore be removed via single point constraints at these grids.  In addition, since the PROD entry has zero torsional constant (field 4 of PROD is blank), there will be no stiffness for global degree of freedom 5 at grids 101 - 601.  Thus, field 7 of the grid 101 - 601 entries have 13456 constrained.  These constraints do not have to appear on the GRID entry, they can be on SPC (or SPC1) entries as well.  Because they appear on the GRID entry these constraints will be used regardless of whether an SPC = SID entry appears in Case Control.  Grid 701, on the other hand, uses coordinate system 13 as its global coordinate system.  Thus, by the same reasoning as above, global degrees of freedom 12456 are taken as permanent single point constraints.

- The connection entries for the rod elements are the six CROD’s whose element IDs are indicated in field 2.  Field 3 (with 16 in it) is the property ID and points to the PROD, ID = 16) for the rod elements properties, which are all the same in this example.  Fields 4 and 5 give the grids to which the elements are attached.

- The PROD 16 entry points to a material entry (ID = 20) in field 3 and gives the rod cross-sectional area in field 4.

- The material properties are defined on the MAT1 with ID = 20.  Only Young’s modulus is needed for this example but a material density of 0.1 is also entered in field 6.

- Case Control had a request for single point constraint set. The SPC entry, with set ID 19, specifies the remaining constraint of zero displacement in global degree of freedom 2 at grid 101.  This could have been included with the constraints specified in field 7 of the GRID 101 entry, in which case the SPC = 19 would not have been needed in Case Control.

- Case Control had a request for load set 191 for subcase 35.  The FORCE Bulk Data entry with ID = 191 is the ID requested for this subcase and defines a 120 lb load at grid 701.  The coordinate system for this load definition is coordinate system 13 (indicated by the 13 in field 4).  Since the components of the load vector are 0., 0., 1. (fields6-8) this indicates a force in the Z13 direction which is along the axis of the rod.

- Case Control also had a request for load set 26 for subcase 8.  As shown above, this loading condition has axial loads on three grid points.  As such, these could have been defined using three FORCE Bulk Data entries, all with set ID = 26.  However, the LOAD (load combining) Bulk Data entry will be used for illustrative purposes.  The LOAD entry has set ID = 26 which is the ID requested for this subcase.  It defines a load that is a linear combination of load sets 39, 5 and 178, where the loads for sets 39, 5 and 178 are specified on the FORCE Bulk Data entries below the LOAD 26 entry.  The linear combination on LOAD 26 is:

- The PARAM GRDPNT 101 requests that the Grid Point Weight Generator calculate the total model mass properties relative to grid point 101.

- The PARAM PRTDOF 1 requests printing of the degree of freedom table.

- The ENDDATA signifies the end of the Bulk Data Deck.

- The remainder of the output for the sample problem is shown on the pages following the ENDDATA

- The next of page lists some informational messages printed out as MYSTRAN executes.

- The degree of freedom table is printed as requested via the Bulk Data PARAM PRTDOF entry.  It shows the degree of freedom numbers for each of the displacement sets and is in internal degree of freedom order.  Note on this listing that the A-set (analysis set) has six degrees of freedom and these are the axial degrees of freedom of the rod at the “free” grids, namely 201 – 701.  Note that for grids 201 – 601, the A-set degree of freedom is in the “2” direction.  This is the global “2” direction for these grids, which is the basic Y0 system.  Note also that grid 701 has its A-set degree of freedom as “3” which, since the global system for this grid is coordinate system 13, is in the Z13 direction

- The Grid Point Weight Generator (GPWG) calculates the model total mass properties and prints them.  In this example problem, 0.1 was the “mass” density on the MAT1 Bulk data entry.  This happens to be the weight density of the aluminum material of which the rod is made.  Thus, the units for the GPWG output are lb.

- The following couple of pages list some informational messages printed out as MYSTRAN executes.

- The remainder of the output shows the items requested in Case Control for each subcase.  The output shows the subcase number at the beginning of each subcases’ output.  The output values are easily verified as being correct with some simple hand calculations.  Note the following:

- Displacement, applied load and constraint force output are for grids and all have headings “T1”, etc, where

T1 is translation in the global X direction of that grid

T2 is translation in the global Y direction of that grid

T3 is translation in the global Z direction of that grid

R1 is rotation about the global X axis

R2 is rotation about the global Y axis

R3 is rotation about the global Z axis

- Grids 201 – 601 have T2 displacements since they use the basic system as global and T2 is in the Y0 direction.  Grid 701, however, uses coordinate system 13 as global and has T3 displacement since T3 is in the Z13 direction

- Element engineering forces and stresses are output in the local element coordinate system for each element.  See Figure 3-2 for the rod element local axes.

- Element node forces are output in the same format as grid point displacements, that is, forces at the grids in global coordinate directions

119150503

MYSTRAN Version  2.06  Jan 19 2006 by Dr Bill Case

>> MYSTRAN BEGIN:  1/19/2006 at 15: 5: 3. 15 The input file is EXAMPLE1.DAT

>> LINK  1 BEGIN

ID ROD SAMPLE PROBLEM FOR USERS MANUAL

SOL 1

CEND

TITLE = ROD WITH AXIAL LOADS IN 2 SUBCASES

ECHO  = UNSORT

SPC   = 19

DISP     = ALL

OLOAD    = ALL

SPCF     = ALL

SUBCASE 35

SUBTITLE = 120 LB LOAD ON GRID 701

ELFORCE  = ALL

STRESS   = ALL

LOAD = 191

SUBCASE 8

SET 98 = 2,5

LOAD = 26

ELFORCE(NODE) = 98

SUBTITLE = 240 LB ON GRID 201 + 150 LB ON GRID 301 + 200 LB ON GRID 401

BEGIN BULK

$

CORD2R  13      0       0.      0.      0.      0.      1.      0.      +CORD13

+CORD13 0.      0.      1.

$

GRID    701             0.      60.     0.      13      12456

GRID    601             0.      50.     0.              13456

GRID    501             0.      40.     0.              13456

GRID    401             0.      30.     0.              13456

GRID    301             0.      20.     0.              13456

GRID    201             0.      10.     0.              13456

GRID    101             0.      0.      0.              13456

$

CROD    1       16      101     201

CROD    2       16      201     301

CROD    3       16      301     401

CROD    4       16      401     501

CROD    5       16      501     601

CROD    6       16      601     701

$

PROD    16      20      .6

$

MAT1    20      1.+7            .33     .1      1.                      +MAT1

*INFORMATION: MAT1 ENTRY       20 HAD FIELD FOR G  BLANK. MYSTRAN CALCULATED G  =  3.759398E+06

+MAT1   10000.  10000.  10000.

$

SPC1    19      2       101

$

FORCE   191     701     13      120.    0.      0.      1.

$

LOAD    26      2.0     4.0     39      3.0     5       1.0     178

FORCE   39      201     0       30.     0.      1.      0.

FORCE   5       301     13      25.     0.      0.      1.

FORCE   178     401     0       100.    0.      1.      0.

$

PARAM   GRDPNT  101

PARAM   PRTDOF  1

DEBUG   200     1

$

ENDDATA

*INFORMATION: SPARSE MATRICES ARE STORED IN SYM    FORMAT

*INFORMATION: BANDIT WAS CALLED TO RESEQUENCE THE GRIDS AND HAS RETURNED WITH ERROR  =        0

*INFORMATION: FILE EXAMPLE1.SEQ

CONTAINING THE BULK DATA SEQGP CARD IMAGES (NEEDED FOR AUTO GRID POINT SEQUENCING REQUESTED BY

THE USER VIA PARAM GRIDSEQ BANDIT  ), DOES NOT EXIST

IT MAY BE THAT BANDIT FOUND THAT NO RESEQUENCING WAS NEEDED OR DUE TO ERROR IN RUNNING BANDIT.

MAKE SURE BANDIT HAS RUN SUCCESSFULLY (CHECK FILE BANDIT.OUT IN THE DIRECTORY WHERE MYSTRAN.EXE RESIDES).

*INFORMATION: SUBR AUTO_SEQ_PROC DID NOT SEQUENCE ALL OF THE        7 GRIDS. ONLY        0 GRIDS WERE SEQUENCED.

MYSTRAN WILL DEFAULT TO A SEQUENCE THAT IS IN GRID NUMERICAL ORDER

DEGREE OF FREEDOM TABLE SORTED ON GRID POINT (TDOF)

(Before any AUTOSPC)

EXTERNAL  INTERNAL                                     DOF NUMBER FOR DISPLACEMENT SET:

GRD-COMP  GRD-COMP ---------------------------------------------------------------------------------------------------------------|

NUMBER    NUMBER          G       M       N      SA      SB      SG      SZ      SE       S       F       O       A       R       L

101-1       1-1       1       0       1       0       0       1       1       0       1       0       0       0       0       0

-2        -2       2       0       2       0       1       0       2       0       2       0       0       0       0       0

-3        -3       3       0       3       0       0       2       3       0       3       0       0       0       0       0

-4        -4       4       0       4       0       0       3       4       0       4       0       0       0       0       0

-5        -5       5       0       5       0       0       4       5       0       5       0       0       0       0       0

-6        -6       6       0       6       0       0       5       6       0       6       0       0       0       0       0

201-1       2-1       7       0       7       0       0       6       7       0       7       0       0       0       0       0

-2        -2       8       0       8       0       0       0       0       0       0       1       0       1       0       1

-3        -3       9       0       9       0       0       7       8       0       8       0       0       0       0       0

-4        -4      10       0      10       0       0       8       9       0       9       0       0       0       0       0

-5        -5      11       0      11       0       0       9      10       0      10       0       0       0       0       0

-6        -6      12       0      12       0       0      10      11       0      11       0       0       0       0       0

301-1       3-1      13       0      13       0       0      11      12       0      12       0       0       0       0       0

-2        -2      14       0      14       0       0       0       0       0       0       2       0       2       0       2

-3        -3      15       0      15       0       0      12      13       0      13       0       0       0       0       0

-4        -4      16       0      16       0       0      13      14       0      14       0       0       0       0       0

-5        -5      17       0      17       0       0      14      15       0      15       0       0       0       0       0

-6        -6      18       0      18       0       0      15      16       0      16       0       0       0       0       0

401-1       4-1      19       0      19       0       0      16      17       0      17       0       0       0       0       0

-2        -2      20       0      20       0       0       0       0       0       0       3       0       3       0       3

-3        -3      21       0      21       0       0      17      18       0      18       0       0       0       0       0

-4        -4      22       0      22       0       0      18      19       0      19       0       0       0       0       0

-5        -5      23       0      23       0       0      19      20       0      20       0       0       0       0       0

-6        -6      24       0      24       0       0      20      21       0      21       0       0       0       0       0

501-1       5-1      25       0      25       0       0      21      22       0      22       0       0       0       0       0

-2        -2      26       0      26       0       0       0       0       0       0       4       0       4       0       4

-3        -3      27       0      27       0       0      22      23       0      23       0       0       0       0       0

-4        -4      28       0      28       0       0      23      24       0      24       0       0       0       0       0

-5        -5      29       0      29       0       0      24      25       0      25       0       0       0       0       0

-6        -6      30       0      30       0       0      25      26       0      26       0       0       0       0       0

601-1       6-1      31       0      31       0       0      26      27       0      27       0       0       0       0       0

-2        -2      32       0      32       0       0       0       0       0       0       5       0       5       0       5

-3        -3      33       0      33       0       0      27      28       0      28       0       0       0       0       0

-4        -4      34       0      34       0       0      28      29       0      29       0       0       0       0       0

-5        -5      35       0      35       0       0      29      30       0      30       0       0       0       0       0

-6        -6      36       0      36       0       0      30      31       0      31       0       0       0       0       0

701-1       7-1      37       0      37       0       0      31      32       0      32       0       0       0       0       0

-2        -2      38       0      38       0       0      32      33       0      33       0       0       0       0       0

-3        -3      39       0      39       0       0       0       0       0       0       6       0       6       0       6

-4        -4      40       0      40       0       0      33      34       0      34       0       0       0       0       0

-5        -5      41       0      41       0       0      34      35       0      35       0       0       0       0       0

-6        -6      42       0      42       0       0      35      36       0      36       0       0       0       0       0

------- ------- ------- ------- ------- ------- ------- ------- ------- ------- ------- ------- ------- -------

TOTAL NUMBER OF DOF:      42       0      42       0       1      35      36       0      36       6       0       6

O U T P U T   F R O M   G R I D   P O I N T   W E I G H T   G E N E R A T O R

REFERENCE POINT IS GRID POINT      101

TOTAL MASS =  3.600000E+00

X             Y             Z

C.G. LOCATION :  0.000000E+00  3.000000E+01  0.000000E+00

(RELATIVE TO REFERENCE POINT IN BASIC COORDINATE SYSTEM)

M.O.I. MATRIX - ABOUT REFERENCE POINT IN BASIC COORDINATE SYSTEM

***                                       ***

*  4.380000E+03  0.000000E+00  0.000000E+00 *

*  0.000000E+00  0.000000E+00  0.000000E+00 *

*  0.000000E+00  0.000000E+00  4.380000E+03 *

***                                       ***

M.O.I. MATRIX - ABOUT C.G. IN BASIC COORDINATE SYSTEM

***                                       ***

*  1.140000E+03  0.000000E+00  0.000000E+00 *

*  0.000000E+00  0.000000E+00  0.000000E+00 *

*  0.000000E+00  0.000000E+00  1.140000E+03 *

***                                       ***

M.O.I. MATRIX - ABOUT C.G. IN PRINCIPAL DIRECTIONS

***                                       ***

*  0.000000E+00  0.000000E+00  0.000000E+00 *

*  0.000000E+00  1.140000E+03  0.000000E+00 *

*  0.000000E+00  0.000000E+00  1.140000E+03 *

***                                       ***

TRANSFORMATION FROM BASIC COORDINATES TO PRINCIPAL DIRECTIONS

***                                       ***

*  0.000000E+00  1.000000E+00  0.000000E+00 *

*  1.000000E+00  0.000000E+00  0.000000E+00 *

*  0.000000E+00  0.000000E+00  1.000000E+00 *

***                                       ***

*INFORMATION: LTERM_MGGE ESTIMATE OF THE NUMBER OF NONZEROS IN MASS MATRIX MGGE IS   =          468

*INFORMATION: NUMBER OF NONZERO TERMS IN THE MGG MASS MATRIX IS                      =            7

*INFORMATION: NUMBER OF NONZERO TERMS IN THE MGG MASS MATRIX IS                      =            7

*INFORMATION: MAX NUMBER OF NONZERO TERMS IN A ROW OF THE G-SET MASS MATRIX          =            1

*INFORMATION: LTERM_KGG ESTIMATE OF THE NUMBER OF NONZEROS IN STIFF MATRIX KGG IS    =          468

*INFORMATION: NUMBER OF NONZERO TERMS IN THE KGG STIFFNESS MATRIX IS                 =           13

*INFORMATION: MAX NUMBER OF NONZERO TERMS IN A ROW OF THE G-SET STIFFNESS MATRIX     =            2

*INFORMATION: NUMBER OF GRID POINTS                                                  =            7

*INFORMATION: NUMBER OF G SET DEGREES OF FREEDOM (NDOFG)                             =           42

>> LINK  1 END

>> LINK  2 BEGIN

*INFORMATION: BASED ON PARAMETER AUTOSPC_NSET =  1 MYSTRAN IS CHECKING KNN TO SEE IF THERE ARE NULL ROWS THAT SHOULD BE AUTOSPC'd

*INFORMATION: MYSTRAN FOUND NO N-SET DOF's THAT WERE SINGULAR AND THAT WERE NOT ALREADY MEMBERS OF THE S-SET

*INFORMATION: AUTOSPC Summary, Overall: after identification of all AUTOSPC's

AUTOSPC_RAT = 1.000000E-06

Number of DOF's identified for AUTOSPC in component  1         =            0

Number of DOF's identified for AUTOSPC in component  2         =            0

Number of DOF's identified for AUTOSPC in component  3         =            0

Number of DOF's identified for AUTOSPC in component  4         =            0

Number of DOF's identified for AUTOSPC in component  5         =            0

Number of DOF's identified for AUTOSPC in component  6         =            0

------------

Total number of DOF's identified overall                       =            0

*INFORMATION: NUMBER OF  M SET DEGREES OF FREEDOM (NDOFM)                            =            0

*INFORMATION: NUMBER OF  N SET DEGREES OF FREEDOM (NDOFN)                            =           42

*INFORMATION: NUMBER OF  S SET DEGREES OF FREEDOM (NDOFS)                            =           36

*INFORMATION: NUMBER OF SA SET DEGREES OF FREEDOM (NDOFSA)                           =            0

*INFORMATION: NUMBER OF  F SET DEGREES OF FREEDOM (NDOFF)                            =            6

*INFORMATION: NUMBER OF  O SET DEGREES OF FREEDOM (NDOFO)                            =            0

*INFORMATION: NUMBER OF  A SET DEGREES OF FREEDOM (NDOFA)                            =            6

*INFORMATION: NUMBER OF  R SET DEGREES OF FREEDOM (NDOFR)                            =            0

*INFORMATION: NUMBER OF  L SET DEGREES OF FREEDOM (NDOFL)                            =            6

>> LINK  2 END

>> LINK  3 BEGIN

*INFORMATION: NUMBER OF SUPERDIAGONALS IN THE UPPER TRIANGLE OF MATRIX        KLL   =            1

*INFORMATION: MAXIMUM DIAGONAL TERM IN MATRIX         KLL                            =  1.200000E+06 Occurs in row/col no.        1

*INFORMATION: MINIMUM DIAGONAL TERM IN MATRIX         KLL                            =  6.000000E+05 Occurs in row/col no.        6

*INFORMATION: RATIO OF MAX TO MIN DIAGONALS IN MATRIX         KLL                    =  2.000000E+00

*INFORMATION: MAX RATIO OF MATRIX DIAGONAL TO FACTOR DIAGONAL FOR MATRIX         KLL =  1.897367E+03 Occurs in row/col no.        6

*INFORMATION: FOR INTERNAL SUBCASE NUMBER        1 EPSILON ERROR ESTIMATE            =  1.421085E-15 Based on U'*(K*U - P)/(U'*P)

*INFORMATION: FOR INTERNAL SUBCASE NUMBER        2 EPSILON ERROR ESTIMATE            =  1.104361E-15 Based on U'*(K*U - P)/(U'*P)

>> LINK  3 END

>> LINK  5 BEGIN

>> LINK  5 END

>> LINK  9 BEGIN

SUBCASE       35

ROD WITH AXIAL LOADS IN 2 SUBCASES

120 LB LOAD ON GRID 701

D I S P L A C E M E N T S

(in global coordinate system at each grid)

GRID     COORD      T1            T2            T3            R1            R2            R3

SYS

101        0  0.000000E+00  0.000000E+00  0.000000E+00  0.000000E+00  0.000000E+00  0.000000E+00

201        0  0.000000E+00  2.000000E-04  0.000000E+00  0.000000E+00  0.000000E+00  0.000000E+00

301        0  0.000000E+00  4.000000E-04  0.000000E+00  0.000000E+00  0.000000E+00  0.000000E+00

401        0  0.000000E+00  6.000000E-04  0.000000E+00  0.000000E+00  0.000000E+00  0.000000E+00

501        0  0.000000E+00  8.000000E-04  0.000000E+00  0.000000E+00  0.000000E+00  0.000000E+00

601        0  0.000000E+00  1.000000E-03  0.000000E+00  0.000000E+00  0.000000E+00  0.000000E+00

701       13  0.000000E+00  0.000000E+00  1.200000E-03  0.000000E+00  0.000000E+00  0.000000E+00

------------- ------------- ------------- ------------- ------------- -------------

MAX (for output set):    0.000000E+00  1.000000E-03  1.200000E-03  0.000000E+00  0.000000E+00  0.000000E+00

MIN (for output set):    0.000000E+00  0.000000E+00  0.000000E+00  0.000000E+00  0.000000E+00  0.000000E+00

ABS (for output set):    0.000000E+00  1.000000E-03  1.200000E-03  0.000000E+00  0.000000E+00  0.000000E+00

SUBCASE       35

ROD WITH AXIAL LOADS IN 2 SUBCASES

120 LB LOAD ON GRID 701

A P P L I E D    F O R C E S

(in global coordinate system at each grid)

GRID     COORD      T1            T2            T3            R1            R2            R3

SYS

101        0  0.000000E+00  0.000000E+00  0.000000E+00  0.000000E+00  0.000000E+00  0.000000E+00

201        0  0.000000E+00  0.000000E+00  0.000000E+00  0.000000E+00  0.000000E+00  0.000000E+00

301        0  0.000000E+00  0.000000E+00  0.000000E+00  0.000000E+00  0.000000E+00  0.000000E+00

401        0  0.000000E+00  0.000000E+00  0.000000E+00  0.000000E+00  0.000000E+00  0.000000E+00

501        0  0.000000E+00  0.000000E+00  0.000000E+00  0.000000E+00  0.000000E+00  0.000000E+00

601        0  0.000000E+00  0.000000E+00  0.000000E+00  0.000000E+00  0.000000E+00  0.000000E+00

701       13  0.000000E+00  0.000000E+00  1.200000E+02  0.000000E+00  0.000000E+00  0.000000E+00

------------- ------------- ------------- ------------- ------------- -------------

MAX (for output set):    0.000000E+00  0.000000E+00  1.200000E+02  0.000000E+00  0.000000E+00  0.000000E+00

MIN (for output set):    0.000000E+00  0.000000E+00  0.000000E+00  0.000000E+00  0.000000E+00  0.000000E+00

ABS (for output set):    0.000000E+00  0.000000E+00  1.200000E+02  0.000000E+00  0.000000E+00  0.000000E+00

------------- ------------- ------------- ------------- ------------- -------------

APPLIED FORCE TOTALS: not printed since all grids do not have the same global coordinate system

SUBCASE       35

ROD WITH AXIAL LOADS IN 2 SUBCASES

120 LB LOAD ON GRID 701

S P C   F O R C E S

(in global coordinate system at each grid)

GRID     COORD      T1            T2            T3            R1            R2            R3

SYS

101        0  0.000000E+00 -1.200000E+02  0.000000E+00  0.000000E+00  0.000000E+00  0.000000E+00

201        0  0.000000E+00  0.000000E+00  0.000000E+00  0.000000E+00  0.000000E+00  0.000000E+00

301        0  0.000000E+00  0.000000E+00  0.000000E+00  0.000000E+00  0.000000E+00  0.000000E+00

401        0  0.000000E+00  0.000000E+00  0.000000E+00  0.000000E+00  0.000000E+00  0.000000E+00

501        0  0.000000E+00  0.000000E+00  0.000000E+00  0.000000E+00  0.000000E+00  0.000000E+00

601        0  0.000000E+00  0.000000E+00  0.000000E+00  0.000000E+00  0.000000E+00  0.000000E+00

701       13  0.000000E+00  0.000000E+00  0.000000E+00  0.000000E+00  0.000000E+00  0.000000E+00

------------- ------------- ------------- ------------- ------------- -------------

MAX (for output set):    0.000000E+00  0.000000E+00  0.000000E+00  0.000000E+00  0.000000E+00  0.000000E+00

MIN (for output set):    0.000000E+00 -1.200000E+02  0.000000E+00  0.000000E+00  0.000000E+00  0.000000E+00

ABS (for output set):    0.000000E+00  1.200000E+02  0.000000E+00  0.000000E+00  0.000000E+00  0.000000E+00

------------- ------------- ------------- ------------- ------------- -------------

SPC FORCE TOTALS: not printed since all grids do not have the same global coordinate system

SUBCASE       35

ROD WITH AXIAL LOADS IN 2 SUBCASES

120 LB LOAD ON GRID 701

E L E M E N T   E N G I N E E R I N G   F O R C E S

F O R   E L E M E N T   T Y P E   R O D

Element     Axial         Torque     Element     Axial         Torque     Element     Axial         Torque

ID       Force                       ID       Force                       ID       Force

1  1.200000E+02  0.000000E+00        2  1.200000E+02  0.000000E+00        3  1.200000E+02  0.000000E+00

4  1.200000E+02  0.000000E+00        5  1.200000E+02  0.000000E+00        6  1.200000E+02  0.000000E+00

SUBCASE       35

ROD WITH AXIAL LOADS IN 2 SUBCASES

120 LB LOAD ON GRID 701

E L E M E N T   S T R E S S E S   I N   L O C A L   E L E M E N T   C O O R D I N A T E   S Y S T E M

F O R   E L E M E N T   T Y P E   R O D

Element     Axial           Safety     Torsional         Safety    Element     Axial       Safety     Torsional     Safety

ID       Stress          Margin       Stress          Margin       ID       Stress      Margin       Stress      Margin

1  2.000000E+02  4.90E+01   0.000000E+00                  2  2.000000E+02  4.90E+01   0.000000E+00

3  2.000000E+02  4.90E+01   0.000000E+00                  4  2.000000E+02  4.90E+01   0.000000E+00

5  2.000000E+02  4.90E+01   0.000000E+00                  6  2.000000E+02  4.90E+01   0.000000E+00

SUBCASE        8

ROD WITH AXIAL LOADS IN 2 SUBCASES

240 LB ON GRID 201 + 150 LB ON GRID 301 + 200 LB ON GRID 401

D I S P L A C E M E N T S

(in global coordinate system at each grid)

GRID     COORD      T1            T2            T3            R1            R2            R3

SYS

101        0  0.000000E+00  0.000000E+00  0.000000E+00  0.000000E+00  0.000000E+00  0.000000E+00

201        0  0.000000E+00  9.833333E-04  0.000000E+00  0.000000E+00  0.000000E+00  0.000000E+00

301        0  0.000000E+00  1.566667E-03  0.000000E+00  0.000000E+00  0.000000E+00  0.000000E+00

401        0  0.000000E+00  1.900000E-03  0.000000E+00  0.000000E+00  0.000000E+00  0.000000E+00

501        0  0.000000E+00  1.900000E-03  0.000000E+00  0.000000E+00  0.000000E+00  0.000000E+00

601        0  0.000000E+00  1.900000E-03  0.000000E+00  0.000000E+00  0.000000E+00  0.000000E+00

701       13  0.000000E+00  0.000000E+00  1.900000E-03  0.000000E+00  0.000000E+00  0.000000E+00

------------- ------------- ------------- ------------- ------------- -------------

MAX (for output set):    0.000000E+00  1.900000E-03  1.900000E-03  0.000000E+00  0.000000E+00  0.000000E+00

MIN (for output set):    0.000000E+00  0.000000E+00  0.000000E+00  0.000000E+00  0.000000E+00  0.000000E+00

ABS (for output set):    0.000000E+00  1.900000E-03  1.900000E-03  0.000000E+00  0.000000E+00  0.000000E+00

SUBCASE        8

ROD WITH AXIAL LOADS IN 2 SUBCASES

240 LB ON GRID 201 + 150 LB ON GRID 301 + 200 LB ON GRID 401

A P P L I E D    F O R C E S

(in global coordinate system at each grid)

GRID     COORD      T1            T2            T3            R1            R2            R3

SYS

101        0  0.000000E+00  0.000000E+00  0.000000E+00  0.000000E+00  0.000000E+00  0.000000E+00

201        0  0.000000E+00  2.400000E+02  0.000000E+00  0.000000E+00  0.000000E+00  0.000000E+00

301        0  0.000000E+00  1.500000E+02  0.000000E+00  0.000000E+00  0.000000E+00  0.000000E+00

401        0  0.000000E+00  2.000000E+02  0.000000E+00  0.000000E+00  0.000000E+00  0.000000E+00

501        0  0.000000E+00  0.000000E+00  0.000000E+00  0.000000E+00  0.000000E+00  0.000000E+00

601        0  0.000000E+00  0.000000E+00  0.000000E+00  0.000000E+00  0.000000E+00  0.000000E+00

701       13  0.000000E+00  0.000000E+00  0.000000E+00  0.000000E+00  0.000000E+00  0.000000E+00

------------- ------------- ------------- ------------- ------------- -------------

MAX (for output set):    0.000000E+00  2.400000E+02  0.000000E+00  0.000000E+00  0.000000E+00  0.000000E+00

MIN (for output set):    0.000000E+00  0.000000E+00  0.000000E+00  0.000000E+00  0.000000E+00  0.000000E+00

ABS (for output set):    0.000000E+00  2.400000E+02  0.000000E+00  0.000000E+00  0.000000E+00  0.000000E+00

------------- ------------- ------------- ------------- ------------- -------------

APPLIED FORCE TOTALS: not printed since all grids do not have the same global coordinate system

SUBCASE        8

ROD WITH AXIAL LOADS IN 2 SUBCASES

240 LB ON GRID 201 + 150 LB ON GRID 301 + 200 LB ON GRID 401

S P C   F O R C E S

(in global coordinate system at each grid)

GRID     COORD      T1            T2            T3            R1            R2            R3

SYS

101        0  0.000000E+00 -5.900000E+02  0.000000E+00  0.000000E+00  0.000000E+00  0.000000E+00

201        0  0.000000E+00  0.000000E+00  0.000000E+00  0.000000E+00  0.000000E+00  0.000000E+00

301        0  0.000000E+00  0.000000E+00  0.000000E+00  0.000000E+00  0.000000E+00  0.000000E+00

401        0  0.000000E+00  0.000000E+00  0.000000E+00  0.000000E+00  0.000000E+00  0.000000E+00

501        0  0.000000E+00  0.000000E+00  0.000000E+00  0.000000E+00  0.000000E+00  0.000000E+00

601        0  0.000000E+00  0.000000E+00  0.000000E+00  0.000000E+00  0.000000E+00  0.000000E+00

701       13  0.000000E+00  0.000000E+00  0.000000E+00  0.000000E+00  0.000000E+00  0.000000E+00

------------- ------------- ------------- ------------- ------------- -------------

MAX (for output set):    0.000000E+00  0.000000E+00  0.000000E+00  0.000000E+00  0.000000E+00  0.000000E+00

MIN (for output set):    0.000000E+00 -5.900000E+02  0.000000E+00  0.000000E+00  0.000000E+00  0.000000E+00

ABS (for output set):    0.000000E+00  5.900000E+02  0.000000E+00  0.000000E+00  0.000000E+00  0.000000E+00

------------- ------------- ------------- ------------- ------------- -------------

SPC FORCE TOTALS: not printed since all grids do not have the same global coordinate system

SUBCASE        8

ROD WITH AXIAL LOADS IN 2 SUBCASES

240 LB ON GRID 201 + 150 LB ON GRID 301 + 200 LB ON GRID 401

E L E M   N O D A L   F O R C E S   I N   G L O B A L   C O O R D S

F O R   E L E M E N T   T Y P E   R O D

Element     Grid         T1            T2            T3            R1            R2            R3

ID      Point

2      201  0.000000E+00 -3.500000E+02  0.000000E+00  0.000000E+00  0.000000E+00  0.000000E+00

301  0.000000E+00  3.500000E+02  0.000000E+00  0.000000E+00  0.000000E+00  0.000000E+00

5      501  0.000000E+00 -2.273737E-13  0.000000E+00  0.000000E+00  0.000000E+00  0.000000E+00

601  0.000000E+00  2.273737E-13  0.000000E+00  0.000000E+00  0.000000E+00  0.000000E+00

------------- ------------- ------------- ------------- ------------- -------------

MAX (for output set):    0.000000E+00  3.500000E+02  0.000000E+00  0.000000E+00  0.000000E+00  0.000000E+00

MIN (for output set):    0.000000E+00 -3.500000E+02  0.000000E+00  0.000000E+00  0.000000E+00  0.000000E+00

ABS (for output set):    0.000000E+00  3.500000E+02  0.000000E+00  0.000000E+00  0.000000E+00  0.000000E+00

>> LINK  9 END

>> MYSTRAN END  :  1/19/2006 at 15: 5: 3.8. The output file is:

EXAMPLE1.F06

MYSTRAN terminated normally. Total CPU time =  1.56E-01 seconds

# Equations for the reduction of the G-set to the A-set and solution for displacements and constraint forces

## Introduction

As discussed in Section 3.6, MYSTRAN builds the original stiffness and mass matrices based on the G-set, which has 6 degrees of freedom per grid specified in the Bulk Data deck.  The stiffness matrix is by definition singular as, at this point, there have been no constraints imposed.  There are two type of constraints MYSTRAN allows; single point constraints and multi-point constraints as discussed earlier in this manual.  In order to apply boundary conditions that restrain the model from rigid body motion, single point constraints must be used.  Multi-point constraints (using rigid elements or Bulk Data MPC entries) are used to express some degrees of freedom (DOF’s) of the model as being rigidly restrained to some other DOF’s.  Thus, MYSTRAN must reduce the G-set stiffness, mass, and loads to the independent A-set DOF’s

The discussion below shows the process that MYSTRAN uses to solve for the displacements and constraint forces by going through a systematic reduction of the G-set to the N-set then to the F-set and finally to the L-set which represent the independent DOF’s.  These equations can then be solved for the L-set DOF’s.  The other DOF displacements, as well as constraint forces, can then be recovered.  Element forces and stresses are obtained from the displacements as discussed in Appendix C.  The process in this appendix uses the displacement set notation developed in Section 3.6 which should be reviewed prior to this section.  In general, the matrix notation used in this development is such that the matrix subscripts describe the matrix size.  Thus, KGG is a matrix which has G rows and columns, RCG is a matrix that has C rows and G columns and RTCG is the transpose of RCG and has G rows and C columns.  If a matrix has only one column, it would exhibit only one subscript, as in YS which is an S x 1 matrix of single point constraint values

## Reduction of the G-set to the N-set

In terms of this G-set, the equations of motion for the structure can be written as:

In the first of equations 8.1 MGG  is the G-set mass matrix, KGG is the G-set stiffness matrix, UG are the G-set displacements, PG are the applied loads on the G-set DOF’s and qC are the independent, generalized, constraint forces (due to single and multi-point constraints).  The second of 8.1 expresses the constraints (both single and multi-point constraints) wherein C is the number of constraint equations, RCG is a constraint coefficient matrix and YC is a vector of constraint values.  For example, if all of the constraints were single point constraints, then all of the coefficients in any one row of RCG would be zero except for one unity value.  In addition, if all of these single point constraints were for DOF’s that are grounded, then all of the YC values would be zero and these single point constraints would all have the form of ui = 0.

The unknowns in 8.1 are the UG displacements and the qC generalized constraint forces and there are G+C equations to solve for these unknowns.  As will be explained later, direct solution of the qC constraint forces will not be made.

The qC generalized forces of constraint do not necessarily have any physical meaning.  Rather, the G-set nodal forces of constraint are of interest and are expressed in terms of the qC as:

In order to reduce 8.1 the G-set is partitioned into the N and M-sets, where the M DOF’s are to be eliminated using the multi-point constraints (from rigid elements as well as MPC Bulk Data entries defined by the user in the input data deck).  The UN are the remainder of the DOF’s in the G-set.  Thus, write UG as:

The number of constraints is C which is equal to M+S (where S is the number of DOF’s in the S set).  Thus, partition qC and YC as:

0M is a column vector of M zeros.  That is, only the S-set can have nonzero constraint values.

With the second of 8.4 in mind, partition the second of equations 8.1 using 8.3 as:

The 0SM partition is an S x M matrix of zero’s.  This is required by the form of the single point constraint equations which are all of the form ui = Yi where Yi is a constant (zero or some enforced displacement value).

Using 8.3, partition the first of equations 8.1 as:

The bars over the N-set mass, stiffness and loads matrices are used for convenience to distinguish these terms from those that will result from the reduction of the G-set to the N-set.  From the second of the constraint equations in 8.5 solve for UM in terms of UN:

where

Using 8.7, equation 8.3 can be written as:

where INN is an identity matrix of size N.

Substitute 8.9 into 8.6 and premultiply the result by the transpose of the coefficient matrix in 8.9.  The result can be written as:

where:

MNN , KNN and PN are the reduced N-set mass stiffness and loads.  Note that PN is not the set of applied loads on the N-set if there are applied loads on the M-set as expressed by the second of equations 8.11 ( are the applied loads on the N set).

In addition, the second term in the square brackets in 8.10 is zero by the definition of GMN in 8.8 so that 8.10 and 8.5 can be written as:

## Reduction of the N-set to the F-set

The N-set can now be partitioned into the F and S-sets where the S DOF’s are to be eliminated using the single point constraints identified by the user in the input data deck.  The F-set are the remainder of the DOF’s in the N-set and are known as the “free” DOF’s (i.e. those that have no constraints imposed on them).  Thus, partition UN into UF and US:

Rewrite equation 8.5 in terms of the F, S and M-sets with the restriction that the single point constraints are of the form ui = Yi where Yi is a constant (zero or some enforced displacement value), using:

where OSF is an S x F matrix of zeros and ISS is an S size identity matrix.  Equation 8.5 can be written as:

Substitute 8.13 and the first of 8.14 into 8.12 and partition the mass, stiffness and load matrices into the F and S-sets to get:

Note that 0SF is the transpose of 0FS and is an S x F matrix of zero’s. From the first of 8.15 it is seen that the single point constraints are of the form:

where YS is a column matrix of known constant displacement values (either zero or some enforced displacement).  This agrees with the single point constraint form discussed above; that is, single point constraints express one DOF as being equal to a constant.

Substituting 8.17 into the first of 8.16 results in the equations for the F-set displacements:

where

At this point the F-set equations in 8.18 can be solved for since there are F unknowns and F equations with which to solve for them.  However, MYSTRAN also allows for a Guyan reduction which, although not generally used in static analysis, may be relevant for eigenvalue analysis.  In eigenvalue analyses by the GIV method (see EIGR Bulk Data entry), the mass matrix must be nonsingular.  In a situation where the model has no mass for the rotational DOF’s, the mass matrix would be singular.  Guyan reduction to statically condense massless DOF’s will result in a nonsingular mass matrix.  Thus, if the user identifies an O set, there is a further reduction; that from the F-set to the A-set

## Reduction of the F-set to the A-set

The F-set is partitioned into the A and O-sets where the O DOF’s are to be eliminated using Guyan reduction identified by the user either through the use of ASET/ASET1 or OMIT/OMIT1 entries in the input data deck.  The A-set are the remainder of the DOF’s in the F-set and are known as the “analysis” DOF’s.  Thus, partition UF into UA and UO:

Substitute 8.20 into 8.18 and partition the stiffness and load matrices into the A and O-sets to get:

Guyan reduction is only exact, in general, for a statics problem.  In a dynamic problem it is only exact if there is no mass on the O-set.  In order to explain the Guyan reduction, consider equation 8.21 for a statics problem:

In a static analysis (=0)  the second of 8.21 can be used to get:

From the 2nd of 8.22 we can solve for  in terms of .  We can then write:

The first part of the first equation in 8.23 suggests the possibility of using:

Using 8.24 in 8.22 and premutiplying by the  transpose of the coefficient matrix in 8.24 yields:

Which is exactly what would have been found if 8.23 had been substituted into 8.22 for .

Equation 8.24 to can be used as a way to eliminate the O-set degrees of freedom for the dynamic system of equations in 8.21.  This would be an approximation unless there was no mass associated with the O-set degrees of freedom and is the classic Guyan reduction approximation made in dynamic analyses in which the O-set is eliminated by static condensation (i.e. using the  in equation 8.23).  Using 8.24 in 8.21 yields

where:

Now, equation 8.27 can be solved for the A-set DOF displacements.  The process of recovering the displacements of the O, S and M-set displacements is accomplished by reversing the process we just went through in the reduction.  First, the O set displacements are recovered using 8.23.  The combination of the A and O-sets yields the F-set.  The S-set is given by 8.17.  The combination of the F and S-sets yields the N-set.  The M-set is recovered from the N-set by 8.7 and the combination of the N and M-sets yield the complete model displacements in the G-set.

## Reduction of the A-set to the L-set

The A-set is partitioned into the L and R-sets where the R DOF’s are boundary DOF’s where one substructure attaches to another in Craig-Bampton (CB) analyses.  The modal properties of the substructure in CB analysis are fixed boundary modes so that, for the modal portion of CB, the R-set are constrained to zero.  The development of the subsequent CB equations of motion in terms of the modal and boundary DOF’s will not be presented here.  See Appendix D and reference 11 for a complete discussion of CB analyses.  For other analyses there is no R-set so that the L set is the same as the A set for solution of the independent degrees of freedom

## Solution for constraint forces

The constraint forces are recovered as follows.  Rewrite 8.2 by partitioning QG into QF, QN and QM and partitioning qC into qS and qM. Using the coefficient matrix in 8.15 for RCG we get, for QG:

As discussed earlier, the distinction between the q and Q is that the former are generalized forces of constraint and the later are physical constraint forces on the DOF’s of the model.  It is the Q constraint forces that are of interest.

Rewrite 8.28 as:

where 0F and 0M are null column matrices of size F and M.

Equation 8.29 can be written as:

The first term in 8.30 represents the forces of single point constraint and the second the forces of multi-point constraint.  Comparing 8.29 and 8.30:

From the first of 8.31 it is seen that the grid point SPC constraint forces are equal to the generalized qS forces.  Using 8.17 and the second of 8.16 (keeping in mind that the derivatives of the S-set degrees of freedom are zero due to 8.17) the qS, or QS is:

Thus, there are SPC forces only on the S-set DOF’s

From the second of 8.31 and using 8.14 it is seen that the MPC forces can be written as:

From 8.7 and the second of 8.6, solve for qM :

Substituting 8.34 into 8.33 yields:

Using 8.8 this becomes:

This can also be written as:

There are MPC forces on the N-set (which includes the F and S-sets) as well as on the M-set.  Equations 8.32 and 8.36 (or 8.37) are used to determine the constraint forces once the UG are found.

This completes the derivation of the solution for the G-set displacements and the constraint forces.  However, it is of interest to demonstrate that the constraint forces satisfy the principal of virtual work (that is, constraint forces do no virtual work).

Let WC be the work done by the constraint forces and  the virtual work done by the constraint forces.  Write  as:

The virtual work of the constraint forces is equal to the constraint forces moving through a virtual displacement, .  Thus:

By virtue of 8.17:

That is, the virtual displacements of the S-set are zero since YS contains specified values (zero or some enforced displacement).  Therefore:

Thus  must also be zero by virtue of the first of 8.38. This virtual work of the MPC forces can be written as a combination of the virtual work of the MPC forces on the N and M-sets as follows:

Using 8.7 this can be written as:

using 8-41:

Since the virtual displacements of the N-set are not necessarily zero this requires that:

This agrees with 8.36.  Thus, the constraint forces developed above are consistent with the principal of virtual work.

# Equations for Element Stress/Strain Recovery

## General discussion

For the 2D plate elements and 3D solid elements arrays called STRAIN and STRESS are calculated for each element. For 1D elements.. like the rod and beam. only the STRESS array is calculated.  Both arrays STRAIN and STRESS can contain up to 9 rows and there is one of each these calculated for every subcase.  The STRAIN and STRESS arrays are further subdivided as shown below:

where  STRAINi and STRESSi each have 3 rows

Ue are the displacements of the nodes of the element in the local element coordinate system (see Figures 3-2 through 3-6 in the main body of this manual) and are obtained from the G-set displacements, the solution for which is discussed in Appendix B.  These G-set displacements for the nodes of an element are transformed to the local element coordinate system to obtain Ue  which has a number of rows equal to 6n where n is the number of nodes for the element (e.g. n=4 for a quadrilateral plate element).  There is one Ue for each subcase in the solution.  The BEi arrays each have 3 rows and 6n columns  and are based on the strain-displacement relationships for individual elements.  The SEi are equal to material  matrices times the BEi. The STEi arrays contain the thermal stress effects, if there are any, and have 3 rows and as many columns as there are thermal subcases..  That is, if the input data deck has 5 subcases and two of these have thermal loads, then STEi will have only 2 columns while Ue will have 5 columns.  If a user outputs the SEi and STEi arrays, it is their responsibility to keep track of which subcases the columns of STEi belong.  MYSTRAN does this internally for its stress output calculations.

The following sections show what is contained in arrays STRESSi for each of the element types.  In that manner, it will be obvious how MYSTRAN uses the SEi and STEi arrays, generated internally in MYSTRAN, to obtain stresses.  If desired, they are available to be output to a text or unformatted binary file through use of the Case Control entry ELDATA.  They need not be output for the user to obtain element stresses, however, which are available in the normal text output file through use of the Case Control entry STRESS.

## Rod element

The rod geometry and loading is shown in Figure 3-2 in the main body of this manual.  It is a very simple element and has only two stresses that can be output: the axial stress and the torsional stress. It only uses the first 2 rows of array STRESS1 with row 1 being the axial stress and row 2 the torsional stress.  Array STRESS1 is:

As an example of what is in arrays SE1 and STE1 for a simple element, the arrays are shown below for this rod element.  More complicated elements won’t have a simple closed form for these matrices and will not be shown.

Array SE1 for the rod element is:

E and G are Young’s modulus and shear modulus from the Bulk Data material entry for the element, L is the element length and C is the torsional stress recovery coefficient from a PROD entry.

Array STE1 would have the following column for each subcase that has a thermal load:

and Tref are the coefficient of thermal expansion and reference temperature from the material Bulk Data entry for the element and  is the average element temperature for the thermal subcase.

## Bar element

The bar element geometry and loading is shown in Figures 3-3 and 3-4 in the main body of this manual.  For the bar element, array STRESS uses all 3 rows of STRESS1 and STRESS2.  The first row of STRESS1 contains the actual axial stress in the bar and the third row of STRESS2 contains the actual torsional stress. The second and third rows of STRESS1 and the first two rows of STRESS2 are not actual stress values.  Rather, they are the four independent parameters needed to determine the bending stresses at points in the bar cross-section.  Thus:

and

This can be put into the form of equation 9.2 as:

Kaa and Kab are 6x6 partitions from the 1st 6 rows of  the bar element stiffness matrix and B1, B2 and

are matrices of element properties as shown below:

with the following bar properties:

Stresses due to bending (i.e. not including axial stress at the neutral axis) at ends a and b of the bar element are obtained from:

where  are the bending stresses at ends a and b of the bar and  are the coordinates of a point on the bar cross section as measured in the local element coordinate system (see Figure 3-3 in the main body of this manual).  It should be noted that temperature distributions through the depth of the bar that are higher order than linear are ignored

## Plate elements

Triangular and quadrilateral plate element geometry, loading and stress conventions are shown in Figures 3-5 and 3-6 in the main body of this manual.  They can use all three of the STRESSi arrays.

### Membrane stresses

STRESS1 contains the membrane stresses (at the plate mid-plane)

This can be put into the form of equation 9.2 as:

Em is the 3x3 membrane material matrix, Bm is the element membrane strain-displacement matrix (developed internally in MYSTRAN),  is the 3x1 vector of coefficients of thermal expansion for the material, T is the element average bulk temperature and  is the reference temperature for the element material.

### Bending stresses

STRESS2, times a fiber distance, contains the stresses due to bending, where:

This can be put into the form of equation 9.2 as:

Eb is the 3x3 bending material matrix, Bb is the element bending strain-displacement matrix (developed internally in MYSTRAN),  is the 3x1 vector of coefficients of thermal expansion for the material and  is the temperature gradient through the thickness of the plate element.

### Combined membrane and bending stresses

The total bending and in-plane shear stresses at a fiber distance z are obtained from STRESS1 and STRESS2 as:

### Transverse shear stresses

The average transverse shear stresses through the thickness of the plate (for TRIA3 and QUAD4 elements only) are obtained from STRESS3:

This can be put into the form of equation 9.2 as

Es is the 3x3 transverse shear material matrix and Bs is the element transverse shear strain-displacement matrix (developed internally in MYSTRAN).

The transverse shear stresses are not output in the normal output file even if stress output is requested in Case Control. However, the transverse shear stress resultants (integrals of shear stress through thickness) are output if there is a request in Case Control for element engineering forces

## Solid elements

For the 3D solid elements HEXA, PENTA and TETRA arrays STRAIN and STRESS contain only the 6 actual strains and stresses for a 3D solid:

The BE are strain-displacement matrices that are based on the shape functions chosen for the particular 3D solid element. Once the strains have been calculated the stresses are determined from:

ES is the 6x6 material matrix for a solid and ALPT is the thermal distortion portion of the strains. For a homogeneous isotropic material these are:

MYSTRAN does allow anisotropic element properties for solids and, in that case, ES and ALPT are different

# Craig-Bampton Model Generation

## Craig-Bampton Equations of Motion for Substructures

MYSTRAN has the capability to generate Craig-Bampton (CB) models via SOL 31 (or SOL GEN CB MODEL).  This solution sequence calculates the fixed-base modes of a substructure and generates all of the matrices needed to couple the substructure to other CB models.  This appendix describes the Craig-Bampton method and its implementation in MYSTRAN and includes an example problem to explain the input and output for SOL 31.

Craig and Bampton are credited with the first unified approach to modal synthesis, or substructuring for dynamic analysis, using fixed interface flexible modes augmented by boundary constraint modes to describe each substructure.  Their work was a simplification of earlier work by Hurty who first introduced the concept for substructures with redundant boundary degrees of freedom (DOF’s).

In order to explain the Craig-Bampton (CB) method, consider a structure represented by the picture below that is comprised of several (in this case 5) substructures connected at an arbitrary number of points:

Figure 10.1 - Overall Structure Composed of Several Substructures

Each substructure is joined to one or more other substructures at some number of interface, or boundary, DOF’s (indicated by the hatched areas in the above picture.  The complete structure, consisting of the connected substructures, may or may not be restrained from free body motion.   For any one of the substructures ( j = I, II, III,  etc.) the G-set equations of motion (ignoring damping for the moment) are:

In MYSTRAN nomenclature, the G-set is reduced to the A-set by the elimination of the M-set multi-point constraints, the S-set single point constraints and the O-set omitted DOF’s (using OMIT’s or ASET’s).  The A-set DOF’s for this substructure must contain all DOF’s that will be connected to other substructures   The resulting A-set equations of motion (dropping the j superscript notation for each substructure) are:

where the A set matrices are mathematical reductions from the G-set (see Appendix B for details)

Partition 2 into the R-set and L-set, where, the R-set represents the boundary DOF’s in which this substructure connects with other substructures and the L-set are all free interior DOF’s in this substructure

Notice at this point that there remain forces of constraint only at the substructure attach points as the L-set represents all free DOF’s for this substructure.

At this point we can introduce the transformation from the physical displacements  in equation (3) to what are known as the CB DOF’s; namely the flexible mode DOF’s and the boundary (R-set) DOF’s.  In order to show that this is not any further approximation to equation 3, consider the following argument:

1) the  DOF’s are clearly a complete set of DOF’s for the substructure in that, once they are known, the complete g-set DOF’s for this substructure can be determined.

2) similarly, a new set of DOF’s for the substructure,

are a complete set of DOF’s if  are the generalized DOF’s for  flexible modes when

3) Thus we can take to be a linear combination of  and  or:

if we insist that:

a) are shapes when  and  are modal DOF’s.  That is, the columns of  are the flexible modes, , when the boundary is fixed.  The i-th column of the modal matrix  is .

b)  are shapes when .  That is, the columns of are the L-set shapes for unit motions of the R-set when the flexible mode DOF’s are zero.

The  are easy to understand.  They are the eigenvectors resulting from solving an eigenvalue problem from equations 3 with .  This eigenvalue problem would be:

This requires that the determinant of the coefficient matrix on the left side of equation 6 be zero:

The i-th eigenvector,, is then determined by solving the equation:

Solution of equation 8 requires that one element of  be arbitrarily set (the  are shapes and their amplitude does not matter).  Once equation 8 is solved, the modal matrix is:

The can also be explained easily.  As stated above, the are shapes when the flexible mode response is zero.  We can see from equation 5 that a column of represents the displacements at the L-set DOF’s due to motion at one of the R-set DOF’s while all other R-set DOF’s are zero (as well as all ).  We can therefore solve for from equation 3 by taking all applied forces and accelerations equal to zero and solving the statics problem:

where are static displacements of the L-set. From the second row of equation 10, solve for in terms of :

Thus, the CB DOF’s are contained in  (equation 4) and the transformation between and is:

where I is an R x R identity matrix.  Equation 12 can be written as:

is the CB transformation matrix and is of A-set size.  In MYSTRAN this is called matrix PHIXA.  When expanded to G-set size, PHIXA becomes matrix  PHIXG:

Note that when all flexible modes of the substructure are used in  equation 13 is exact.  In practice, all modes are never used since this would defeat the purpose of making the transformation (which is to find a smaller set of DOF’s  which are nonetheless an accurate representation of the A-set).  Substituting equation 13 into equation 2 and premultiplying the result by the transpose of  yields:

where:

and:

are diagonal matrices of generalized maesses and stiffnesses, respectively.

Equations 15 for the i-th substructure can be written as:

The off-diagonal terms in the above stiffness matrix are zero due to the definition of in equation 11.  In addition, matrix in equation 18 is null if the boundary is a determinant interface.  Equations 14 and 18 are the Craig-Bampton equations of motion for the i-th substructure.  The  are due to applied loads on the R and L-set DOF’s (see equation 16) and the are the interface forces where substructures connect. Once the equations are developed for all substructures, the individual substructures can be connected and the resulting equations solved for the combined R-set and N-set DOF’s  for all substructures.  Once this is done, the forces of inter-connection, or substructure interface forces, (that is, the ) can be solved from the individual substructure equations in the top row of equation 18.  Equation 14 is used to obtain displacements for all G-set DOF’s.

Each organization that is developing a substructure in CB format would deliver the above coefficient matrices in equations 14 and 18 to the organization that is doing the combined structure analysis.  In addition, Displacement and Load Transformation Matrices (DTM’s and LTM’s) collectively known as Output Transformation Matrices, (OTM’s), described below, are also delivered as part of the CB model.

## Development of Displ  Output Transformation Matrices (Displ OTM’s)

Typically, a set of displacement output transformation matrices (displ OTM’s, or DTM’s for short), is delivered with a Craig-Bampton model to the organization that will couple all substructures and solve for the primary unknowns ( and ) in order that desired displacements at some of the substructure G-set DOF’s may be obtained along with the coupled solution.

Once the combined structure has been solved for the primary variables, the original  physical DOF’s could be determined from equation 5 and then element forces and stresses could be determined from the  displacements .  This is called recovery of the  DOF’s and element forces and stresses using the Modal Displacement Method (MDM).  However, as is often the case, equations 18 are solved using a severely truncated set of modes for each substructure.  While this may not compromise the accuracy of the solutions for , it could compromise the accuracy of element forces and stresses calculated using displacements determined from equation 5 with the truncated set of modes. In order to avoid this problem, the  DOF’s can be found using the Modal Acceleration Method (MAM), described below.  It should be noted that the MAM described below ignores damping forces so that it is only useful when the damping is small (e.g. less than 10% or so).

From the bottom row of equation 3, solve for  in terms of the other variables in the equation:

Differentiate equation 5 twice and use the result for  in equation 19, to get:

The term  in equation 20. can be written in a form more convenient for calculation.  From equation 8 it can be seen that:

so that

or

where

substitute equation 21 into equation 20 to get:

The various terms in the coefficient matrices in equation 23 are known as Displacement Transformation Matrices (DTM’s).  Equation 23 can be written as:

where

Equations 24 and 25 represent the MAM for recovering displacements for the L-set, for the i-th substructure, once the assembled substructure equations have been solved for the  DOF’s.  Once the L-set displacements have been found, recovery of the remaining displacements in the G-set is accomplished through the transformation matrices used in their elimination from equation 1 (for details see Appendix B).  At the G-set level, equation 24 is:

.

where each of the G-set DTM’s in equation 26 is obtained from the L-set DTM’s in equation 25 through the normal recovery operations to build back up to the G-set from the L-set.  The coefficient matrix in equation 26 that has DTM’s 1 - 3 in it is called matrix PHIZG.  The table below explains the meaning of each of the DTM’s in equation 26:

Table 10.1

| i-th col of: | Represents: |
| --- | --- |
|  | displ’s of G-set due to a unit accel of the i-th interface DOF (all other R, N set DOF’s zero) |
|  | displ’s of G-set due to a unit accel of the i-th flex mode DOF (all other R, N set DOF’s zero) |
|  | displ’s of G-set due to a unit displ of the i-th interface DOF (all other R, N set DOF’s zero) |
|  | displ’s of G-set due to a unit force on the i-th L-set DOF (all other L-set forces zero) |

## Development of Load Output Transformation Matrices (Load OTM’s)

Once the G-set displacements have been found, substructure element forces and stresses, as well as grid point forces, can be recovered and assembled into a Loads Output Transformation Matrix, or Load OTM (more commonly referred to as LTM). There are several types of quantities one may desire in an LTM.  Equations are developed, below, for several types of LTM quantities typically used in CB analyses.

### LTM Terms for Substructure Interface Forces

From the top row of equation 18, the interface forces can be determined once the substructures have been coupled and the  solved.  The interface forces are:

where is an RxR identity matrix.  Equation 27 can be written as:

### LTM Terms for Net cg Loads

Terms can also be included in the overall LTM that will recover what are known as “net” accelerations at the center of gravity (cg) of the CB model.  These are termed Net Load factors (NLF’s) and represent rigid body accelerations of the cg due to the reaction (or interface) forces, .  The development below demonstrates how these are determined.

Define:

Then:

Substitute equation 27 into 30 for :

For rigid body motion:

where  is the 6 x 6  rigid body mass matrix relative to the cg and is equal to:

and  is given in equation 17.  From equations 31 through 33 we can write the cg acceleration net load factors (NLF’s) as:

However, since the columns of  are rigid body modes.  Therefore:

which can be written as:

### LTM Terms for Element Forces and Stresses

In MYSTRAN, element forces and stresses are obtained from the G-set displacement vector and the individual element stiffness matrices.  Equation 26 is the G-set displacement vector:

Thus the columns of each of the DTM’s represents G-set displacements per unit value of one of the variables as described in Table 10.1.  Therefore, each of the DTM’s can be used as if they were a matrix of displacements in calculating element forces and stresses to give:

### LTM Terms for Grid Point Forces due to multi-point constraints (MPC’s)

There are cases in CB analyses in which the forces due to MPC’s are of interest.  As an example, if a user wishes to determine a load in a bolt at an interface between components, it is common to model the bolt as an MPC where two coincident grids are constrained to have the same displacements.  This section develops the equations for determining an LTM for grid point MPC forces.

Equation 1 for the i-th substructure (dropping the superscript-j notation):

As described in section 10.1 the Q constraint forces on the right side of equation 38 are the constraint forces on the S-set SPC DOF’s, the M-set MPC DOF’s and on the R-set boundary DOF’s respectively.  Since all of the boundary DOF’s are contained in the R-set there should be no constraint forces on the S-set.  That is, all S-set DOF’s should be the result of removing singularities and not the result of grounding the model.  With this assumption, as well as the assumption that there are no applied loads on the M-st degrees of freedom the following equation is valid for the MPC forces on the M-set grids:

We want to get 39 in a form like the other LTM’; that is, in terms of .

From equation 26 with applied loads zero:

The g-set DOF vector can also be written using equation 14:

Differentiating twice:

This can also be written as:

Partition the x DOF’s into R and N as in equation 13.   This will require partitioning  into sub-matrices for the R and N also, so that equation 42 can be written as:

.

Substitute equations 40 and 43 into 39 for  and  respectively to get:

We need to express the boundary constraint forces in equation 44 in terms of the vector as we did for the inertia and stiffness terms.  From 28:

Theboundary forces on the R-set can be expanded from the R-set to the G-set by adding zero rows to 45 for the M, S, O-sets (all of the G-set but the R degrees of freedom) to give

where  is expanded to G-set size by addition of zero rows for M, S, O-sets and is expanded from in the same fashion (recall is an R size identity matrix).  Substituting 46 into 44 we get::

is the LTM for MPC forces at grids that have no applied load

## Development of Acceleration Output Transfer Matrices (Accel OTM)

In addition to the displacement and load output transformation matrices (DTM’s and LTM’s) it is common to supply acceleration output transformation matrices (accel OTM’s or ATM’s for short).  From equation 10-12 and differentiating twice we obtain:

ATM is the acceleration transfer matrix.  Notice that the “degrees of freedom” for the ATM are the accelerations of the boundary and modal degrees of freedom whereas all of the other OTM’s have as degrees of freedom:  boundary accelerations, modal accelerations and boundary displacements.  This is due to the use of the modal acceleration method for recovery of displacements and element forces.

## Correspondence between matrix names and CB Equation Variables

The table below shows the correspondence between variables introduced in the above equations and matrix data block names in the DMAP program in Section 10.5.  Any of these may be output in a MYSTRAN CB model generation analysis using the Executive Control entry OUTPUT4.

Table 10-2

Matrices that can be written to OUTPUT4 files

|  | MYSTRAN Matrix Name (OUTPUT4 matrices) | NASTRAN DMAP Name | CB equation variable in Appendix D (where applicable) | Matrix size1 | Partition rows and/or cols |
| --- | --- | --- | --- | --- | --- |
| 1 | CG_LTM |  |  | 6x(2R+N) |  |
| 2 | DLR | DM |  | LxR | rows and cols |
| 3 | EIGEN_VAL | LAMA |  | NxN |  |
| 4 | EIGEN_VEC | PHIG |  | GxN | rows |
| 5 | GEN_MASS | MI |  | Nx1 vector of diag. terms |  |
| 6 | IF_LTM |  |  | Rx(2R+N) | rows |
| 7 | KAA | KAA |  | AxA | rows and cols |
| 8 | KGG | KGG |  | GxG | rows and cols |
| 9 | KLL | KLL |  | LxL | rows and cols |
| 10 | KRL | KLR(t) |  | LxR | rows and cols |
| 11 | KRR | KRR |  | RxR | rows and cols |
| 12 | KRRcb | KBB |  | RxR | rows and cols |
| 13 | KXX | KRRGN |  | (R+N)x(R+N) |  |
| 14 | LTM | LTM | CG_LTM and IF_LTM merged | (6+R)x(2R+N) |  |
| 15 | MCG | RBMCG |  | 6x6 |  |
| 16 | MEFFMASS |  | Modal effective mass | Nx6 |  |
| 17 | MPFACTOR |  | Modal participation factors | Nx6 or NxR |  |
| 18 | MAA |  |  | AxA | rows and cols |
| 19 | MGG |  |  | GxG | rows and cols |
| 20 | MLL | MLL |  | LxL | rows and cols |
| 21 | MRL | MRL |  | RxL | rows and cols |
| 22 | MRN |  |  | RxN | rows |
| 23 | MRR | MRR |  | RxR | rows and cols |

Table 10-2 (con’t)

|  | MYSTRAN Matrix Name (OUTPUT4 matrices) | NASTRAN DMAP Name | CB equation variable in Appendix D (where applicable) | Matrix size | Partition rows and/or cols |
| --- | --- | --- | --- | --- | --- |
| 24 | MRRcb | MBB |  | RxR | rows and cols |
| 25 | MXX | MRRGN |  | (R+N)x(R+N) |  |
| 26 | PA |  | (A-set static reduced loads - only used in statics) |  | Rows |
| 27 | PG |  | (G-set static loads - only used in statics) |  | Rows |
| 28 | PL |  | (L-set static reduced loads - only used in statics) |  | rows |
| 29 | PHIXG | PHIXG |  | Gx(R+N) | rows |
| 30 | PHIZG |  | The G-set displacement transformation matrix is written out in the F06 file under “C B D I S P L A C E M E N T O T M” | Gx(2R+N) | rows |
| 31 | RBM0 |  | Rigid body mass matrix relative to the basic origin | 6x6 |  |
| 32 | TR6_0 | RBR | : rigid body displacement matrix for R-set relative to the model basic coordinate system | Rx6 | rows |
| 33 | TR6_CG | RBRCG | : rigid body displacement matrix for R-set relative to the model CG | Rx6 | rows |

Notes:

- (t) indicates matrix transposition

- Matrix will be singular if there are rotational DOF’s  but no rotational inertia in the R-set, in which case small rotational inertias may have to be added at these DOF’s.

- Matrix is null if the boundary is a determinant set of DOF’s.

- Matrix  is the rigid body mass matrix if the boundary is a determinant set of DOF’s

## Craig-Bampton model generation example problem

The figure below shows a small example problem that is a frame made of CBAR’s that is a substructure assumed to be attached to some other structure in DOF’s 1,2,3 at grids 11 and 13 and in DOF’s 2,3 at grid 12.  The example problem F06 file (with the input echo’d) is shown on the following pages.  This section will discuss the input and output in an effort to explain the Craig-Bampton model generation process.

Equation 10.26 defines the Craig-Bampton degrees of freedom (CB-DOF’s) as Uz which, for this example, consists of the 18 DOF’s:

- 8 boundary acceleration DOF’s,

- 2 modal acceleration DOF’s,  (see EIGRL request for 2 modes to be extracted)

- 8 boundary displacement DOF’s,

Figure 10.2 – Example CB model: CB-EXAMPLE-12b.DAT

Notes on section 10.6.1: CB-EXAMPLE-12b.F06

The echo of the input shows the following salient points for a CB model generation (much like a SOL 3 eigenvalue analysis in terms of input data):

- Executive Control:

  - SOL 31 indicates CB model generation

  - The OUTPUT4 commands show the matrices that will be written in a format the same as NASTRAN OUTPUT4 files.  These matrix data blocks are ones that are listed on Table 10.2 as allowable OUTPUT4 matrices.  Notice that several are written to unit 21 while others are written to unit 22. As explained in section 5.1 of the MYSTRAN Users Reference Manual, unit numbers 21 through 27 are valid for writing OUTPUT4 matrices.

- Case Control:

- METHOD = 1 is to be used for a normal eigenvalue analysis (same as if SOL were 3)

- Outputs (ACCE, DISP, ELFORCE, STRESS) are for Output Transformation Matrices (OTM’s) for the specified sets.  These will be written to the text F06 file.  In addition they will be written to binary files (same name, CB-EXAMPLE-12b) with extension OP8 for the element related OTM;s (ELFORCE, STRESS in this case and OP9 for the grid related OTM’s (ACCE, DISP in this case)

- Bulk Data:

- Shows the model for this example (notice it has mostly CBAR’s but there is also a RBE2)

- Degrees of freedom at the boundary where this substructure attaches to other substructures are defined with the SUPORT Bulk Data entry.  This is the same procedure that is used in CB analyses by the NASTRAN DMAP (Direct Matrix Abstraction Program) method familiar to NASTRAN CB analysts.

- Eigenvalue extraction, EIGRL requesting 2 modes to be extracted

The delineated F06 output begins on the page following the input model echo and shows the following:

- Eigenvalues extracted

- Messages on the matrices requested to be written to OUTPUT4 files

- For the first 3 of the 18 CB_DOF’s in this example the following output (requested in Case Control) is shown (other 15 were left out for clarity):

- Displacement OTM for the requested grids (see Case Control command DISP = 102)

- Element engineering force OTM (see Case Control command ELFORCE = 201)

- Element stress OTM (see Case Control command STRESS = 202)

- Acceleration OTM.  As shown in equation 10.48 the acceleration OTM has columns for  and  but not . For this example, there are 10 columns in the acceleration OTM (8 boundary acceleration DOF’s and 2 modal acceleration DOF’s)

Notes on section 10.6.2: OUTPUT4 matrices written to CB-EXAMPLE-12b.OP1 and OP2

As shown in the Executive Control section of the F06 file in section 10.6.1, there were 3 matrices requested to be written to unit 21 and 4 to unit 22.  These binary files, translated to text, are shown in section 10.6.2.  The number of actual columns for each matrix is indicated in Table 10.2 but only the first 5 of the columns are shown here for the sake of brevity.  These are several of the important CB matrices needed to couple this CB substructure to other substructures in a combined analysis.  The binary OUTPUT4 files are written in the same format as the NASTRAN OUTPUT4 binary files.

Notes on section 10.6.3: Displ and elem force/stress OTM’s written to CB-EXAMPLE-12b.OP1, OP2

Any output requests in Case Control for grid related outputs (e.g. DISPL, ACCEL) and element force/stress outputs (e.g. ELFORCE, STRESS) are written to the text F06 file and also written to OUTPUT4 binary files (automatically; that is, no formal OUTPUT4 request is needed).  The element related OTM’s are always written to a file with the same filename as the F06 file but with extension OP8.  The grid related OTM’s are written to a file with extension OP9.

The first page of section 10.6.3 is a text translation of the element related OTM’s written to file

CB-EXAMPLE-12b.OP8.  The values are the same as was written to the F06 file for element forces and stresses but are also written to binary files in OUTPUT4 format to be used in analyses that couple the CB substructures.  In order to explain the contents of the binary OP8 file, a text file with extension OT8 is also automatically written (provided any Case Control requests are included for element forces/stresses) describing the contents of the OP8 binary file.  This OT8 text file gives an overview of the OP8 binary file and then goes on to describe each row written to the OP8 file.

The next several pages show the same type of information on the grid related OTM’s written to binary file with extension OP9 (with text description in OT9).  Again, this is the grid related outputs requested in Case Control and also written to the F06 text file.

*

### CB-EXAMPLE-12-b.F06

(delineated – some output not included here for the sake of clarity)

1030180330

MYSTRAN Version 3.00   Oct 20 2006 by Dr Bill Case (this TRIAL edition is SP protected)

>> MYSTRAN BEGIN  : 10/30/2006 at 18: 3:30.640 The input file is CB-EXAMPLE-12-b.DAT

>> LINK  1 BEGIN

SOL 31

$

OUTPUT4   CG_LTM   , IF_LTM   ,         ,       ,       //-1/21 $

OUTPUT4   KRRGN    , RBMCG    , MRRGN   ,       , RBRCG //-1/22 $

OUTPUT4   MR       ,          ,         ,       ,       //-1/21 $

CEND

TITLE  = TEST OF CRAIG-BAMPTON SOLUTION

SUBTI  = FRAME USING CBAR's

SPC    = 1

METHOD = 1

ECHO     = UNSORT

$

SET 101  = 32

SET 102  = 22, 32

SET 201  = 211, 212

SET 202  = 201

$

ACCE     = 101

DISP     = 102

ELFORCE  = 201

STRESS   = 202

MEFFMASS = ALL

MPFACTOR = ALL

$

BEGIN BULK

$

EIGRL   1                       2       2       DPB     -1.     MASS

$

EIGR    2        MGIV                   1       24                      +E1

+E1     MASS

GRID    11              0.      0.      0.

GRID    12              100.    0.      0.

GRID    13              50.     0.      50.

GRID    21              0.      100.    0.

GRID    22              100.    100.    0.

GRID    31              50.     50.     0.

GRID    32              50.     50.     0.

$

RBE2    401     31      123456  32

$

$ Frame support bars

$

CBAR    101     1       13      21      0.0     0.5     1.0             +C1

+C1     56      456

CBAR    102     1       13      22      0.0     0.5     1.0             +C2

+C2     56      456

$

$ Edge bars

$

CBAR    201     2       11      21      0.0     0.0     1.0

CBAR    202     2       12      22      0.0     0.0     1.0

CBAR    203     2       11      12      0.0     0.0     1.0

CBAR    204     2       21      22      0.0     0.0     1.0

$

$ Diag bars

$

CBAR    211     3       11      31      0.0     0.0     1.0

CBAR    212     3       12      31      0.0     0.0     1.0

CBAR    213     3       21      31      0.0     0.0     1.0

CBAR    214     3       22      31      0.0     0.0     1.0

$

PBAR    1       1       0.36    0.09    0.09    0.18

PBAR    2       1       0.10    10.0    10.0    20.0

PBAR    3       1       6.0     6.0     6.0     12.0

$

MAT1    1       10.+6           0.3     0.1

*INFORMATION: MAT1 ENTRY        1 HAD FIELD FOR G  BLANK. MYSTRAN CALCULATED G  =  3.846154E+06

$

CONM2   901     11              150.0   0.0     0.0     -5.0

CONM2   902     12              150.0   0.0     0.0     -5.0

CONM2   903     21              150.0   0.0     0.0     -5.0

CONM2   904     22              150.0   0.0     0.0     -5.0

CONM2   905     32              150.0   0.0     0.0     -5.0

$

SPC1    1       456     13

$

$ BOUNDARY DOF'S

$

SUPORT  11      123     12      23      13      123

$

PARAM   WTMASS   .002591

$

ENDDATA

E I G E N V A L U E   A N A L Y S I S   S U M M A R Y   (LANCZOS  Mode 2 DPB     Shift eigen = -1.00E+00)

NUMBER OF EIGENVALUES EXTRACTED  . . . . . .         2

LARGEST OFF-DIAGONAL GENERALIZED MASS TERM    -2.7E-13 (Vecs renormed to 1.0 for gen masses)

. . .         2

MODE PAIR . . . . . . . . . .

. . .         1

NUMBER OF OFF DIAGONAL GENERALIZED MASS

TERMS FAILING CRITERION OF  1.0E-04. . . . .         0

R E A L   E I G E N V A L U E S

MODE  EXTRACTION      EIGENVALUE           RADIANS              CYCLES            GENERALIZED         GENERALIZED

NUMBER   ORDER                                                                        MASS              STIFFNESS

1       1        3.895211E+03        6.241163E+01        9.933119E+00        1.000000E+00        3.895211E+03

2       2        7.011163E+03        8.373269E+01        1.332647E+01        1.000000E+00        7.011163E+03

>> LINK  4 END

>> LINK  6 BEGIN

*INFORMATION: THE FOLLOWING   7 MATRICES WILL BE WRITTEN TO  2 OUTPUT4 FILES IN THE ORDER LISTED BELOW:

OUTPUT4 file on unit  21 has been created as: CB-EXAMPLE-12-b.OP1 and will contain the matrices:

( 1) CG_LTM          :        6 rows and       18 cols   This is MYSTRAN matrix CG_LTM

( 2) IF_LTM          :        8 rows and       18 cols   This is MYSTRAN matrix IF_LTM

( 3) MR              :        8 rows and        8 cols   This is MYSTRAN matrix MRRcb

OUTPUT4 file on unit  22 has been created as: CB-EXAMPLE-12-b.OP2 and will contain the matrices:

( 1) KRRGN           :       10 rows and       10 cols   This is MYSTRAN matrix KXX

( 2) RBMCG           :        6 rows and        6 cols   This is MYSTRAN matrix MCG

( 3) MRRGN           :       10 rows and       10 cols   This is MYSTRAN matrix MXX

( 4) RBRCG           :        8 rows and        6 cols   This is MYSTRAN matrix TR6

>> LINK  6 END

>> LINK  5 BEGIN

>> LINK  5 END

>> LINK  9 BEGIN

OUTPUT FOR CRAIG-BAMPTON DOF        1 OF       18

C B   D I S P L A C E M E N T   O T M

(in global coordinate system at each grid)

GRID     COORD      T1            T2            T3            R1            R2            R3

SYS

22        0 -1.412939E-05  1.622140E-05  8.242222E-05  5.883709E-07 -1.667433E-06  5.125151E-07

32        0  1.051041E-05 -9.465944E-06 -3.182887E-06 -1.086181E-07 -9.450720E-07  2.106009E-07

C B   E L E M E N T   E N G I N E E R I N G   F O R C E   O T M

F O R   E L E M E N T   T Y P E   B A R

Element       Bend-Moment End A           Bend-Moment End B              - Shear -              Axial         Torque

ID       Plane 1       Plane 2       Plane 1       Plane 2      Plane 1       Plane 2        Force

211  2.091876E-01  7.894539E-01  1.515607E+00 -1.439344E+00 -1.847556E-02  3.151997E-02  6.266800E-01  9.672846E-03

212 -1.133151E-01 -1.008960E-02 -1.725401E+00 -6.166148E-02  2.279833E-02  7.293366E-04 -2.953611E-01 -4.720428E-03

C B   E L E M E N T   S T R E S S   O T M   I N   L O C A L   E L E M E N T   C O O R D I N A T E   S Y S T E M

F O R   E L E M E N T   T Y P E   B A R

Element      SA1           SA2           SA3           SA4          Axial         SA-Max        SA-Min      M.S.-T

ID        SB1           SB2           SB3           SB4          Stress        SB-Max        SB-Min      M.S.-C

201  0.000000E+00  0.000000E+00  0.000000E+00  0.000000E+00 -2.748670E+00 -2.748670E+00 -2.748670E+00

0.000000E+00  0.000000E+00  0.000000E+00  0.000000E+00               -2.748670E+00 -2.748670E+00

OUTPUT FOR CRAIG-BAMPTON DOF        2 OF       18

C B   D I S P L A C E M E N T   O T M

(in global coordinate system at each grid)

GRID     COORD      T1            T2            T3            R1            R2            R3

SYS

22        0 -7.600290E-05  8.243595E-05  3.128787E-04  1.925291E-06  2.220055E-06  1.292053E-07

32        0 -5.990878E-05  6.308617E-05  3.224179E-04  3.643362E-06  4.904270E-07  3.218612E-08

C B   E L E M E N T   E N G I N E E R I N G   F O R C E   O T M

F O R   E L E M E N T   T Y P E   B A R

Element       Bend-Moment End A           Bend-Moment End B              - Shear -              Axial         Torque

ID       Plane 1       Plane 2       Plane 1       Plane 2      Plane 1       Plane 2        Force

211  3.640634E+00 -2.875040E+00 -7.752079E+00  4.486528E+00  1.611173E-01 -1.041083E-01  1.906435E+00 -5.333935E-03

212  3.789705E+00  2.992877E+00 -6.061077E+00 -4.713484E+00  1.393111E-01  1.089844E-01  1.808077E+00  5.333935E-03

C B   E L E M E N T   S T R E S S   O T M   I N   L O C A L   E L E M E N T   C O O R D I N A T E   S Y S T E M

F O R   E L E M E N T   T Y P E   B A R

Element      SA1           SA2           SA3           SA4          Axial         SA-Max        SA-Min      M.S.-T

ID        SB1           SB2           SB3           SB4          Stress        SB-Max        SB-Min      M.S.-C

201  0.000000E+00  0.000000E+00  0.000000E+00  0.000000E+00  7.582667E+00  7.582667E+00  7.582667E+00

0.000000E+00  0.000000E+00  0.000000E+00  0.000000E+00                7.582667E+00  7.582667E+00

OUTPUT FOR CRAIG-BAMPTON DOF        3 OF       18

C B   D I S P L A C E M E N T   O T M

(in global coordinate system at each grid)

GRID     COORD      T1            T2            T3            R1            R2            R3

SYS

22        0  3.800145E-05 -4.121798E-05 -1.564393E-04 -9.626456E-07 -1.110028E-06 -6.460267E-08

32        0  2.995439E-05 -3.154308E-05 -1.612090E-04 -1.821681E-06 -2.452135E-07 -1.609306E-08

C B   E L E M E N T   E N G I N E E R I N G   F O R C E   O T M

F O R   E L E M E N T   T Y P E   B A R

Element       Bend-Moment End A           Bend-Moment End B              - Shear -              Axial         Torque

ID       Plane 1       Plane 2       Plane 1       Plane 2      Plane 1       Plane 2        Force

211 -1.820317E+00  1.437520E+00  3.876039E+00 -2.243264E+00 -8.055864E-02  5.205414E-02 -9.532175E-01  2.666968E-03

212 -1.894852E+00 -1.496438E+00  3.030538E+00  2.356742E+00 -6.965554E-02 -5.449220E-02 -9.040385E-01 -2.666968E-03

C B   E L E M E N T   S T R E S S   O T M   I N   L O C A L   E L E M E N T   C O O R D I N A T E   S Y S T E M

F O R   E L E M E N T   T Y P E   B A R

Element      SA1           SA2           SA3           SA4          Axial         SA-Max        SA-Min      M.S.-T

ID        SB1           SB2           SB3           SB4          Stress        SB-Max        SB-Min      M.S.-C

201  0.000000E+00  0.000000E+00  0.000000E+00  0.000000E+00 -3.791334E+00 -3.791334E+00 -3.791334E+00

0.000000E+00  0.000000E+00  0.000000E+00  0.000000E+00               -3.791334E+00 -3.791334E+00

.

.

.

.

.

.

.

.

.

(output for the 4th – 18th CB DOF deleted)

OUTPUT FOR CRAIG-BAMPTON ACCEL OTM COL        1 OF       10

C B   A C C E L E R A T I O N   O T M

(in global coordinate system at each grid)

GRID     COORD      T1            T2            T3            R1            R2            R3

SYS

32        0  2.199853E-02 -2.028331E-02 -1.681579E-02 -3.363157E-04  8.006145E-03  5.254334E-04

OUTPUT FOR CRAIG-BAMPTON ACCEL OTM COL        2 OF       10

C B   A C C E L E R A T I O N   O T M

(in global coordinate system at each grid)

GRID     COORD      T1            T2            T3            R1            R2            R3

SYS

32        0  0.000000E+00  0.000000E+00 -1.000000E+00 -2.000000E-02  0.000000E+00  0.000000E+00

OUTPUT FOR CRAIG-BAMPTON ACCEL OTM COL        3 OF       10

C B   A C C E L E R A T I O N   O T M

(in global coordinate system at each grid)

GRID     COORD      T1            T2            T3            R1            R2            R3

SYS

32        0  0.000000E+00  0.000000E+00  5.000000E-01  1.000000E-02  0.000000E+00  0.000000E+00

.

.

.

.

.

.

.

.

.

(output for the 4th – 10th Accel OTM columns deleted)

M O D A L   P A R T I C I P A T I O N   F A C T O R S

(dimensionless, in coordinate sys        0)

MODE       T1            T2            T3            R1            R2            R3

NUM

1  1.227574E-01 -1.758352E+00  8.791759E-01  1.259087E+00  6.535370E-02 -5.341716E-01

2  6.061630E-01  1.829524E-01 -9.147622E-02 -4.910542E-01 -1.366914E-01 -4.626569E-01

------------------------------------------------------------------------------------------------------------------------------------

E F F E C T I V E   M O D A L   M A S S E S   O R   W E I G H T S

(in coordinate system        0)

Units are same as units for mass input in the Bulk Data Deck

MODE       T1            T2            T3            R1            R2            R3

NUM

1  6.532677E+01  4.179096E+01  4.694259E+02  3.836785E+05  3.287406E+04  3.611917E+02

2  7.948285E+00  9.016521E-01  1.363070E+01  1.674257E+00  6.082279E+05  4.781873E+05

------------- ------------- ------------- ------------- ------------- -------------

Sum all modes:  7.327506E+01  4.269261E+01  4.830566E+02  3.836801E+05  6.411019E+05  4.785485E+05

Total model mass:  9.325238E+02  9.325238E+02  9.325238E+02  4.105260E+06  4.094237E+06  8.139951E+06

Modes % of total mass*:          7.86          4.58         51.80          9.35         15.66          5.88

*If all modes are calculated the % of total mass should be 100% of the free mass (i.e. not counting mass at constrained DOF's).

Percentages are only printed for components that have finite model mass.

-----

>> LINK  9 END

>> MYSTRAN END    : 10/30/2006 at 18: 3:31.562

### OUTPUT4 matrices written to CB-EXAMPLE-12-b.OP1 and OP2

(OUTPUT4 matrices requested in Exec Control)

OUTPUT4 matrices requested in Exec Control to be written to file CB-EXAMPLE-12-b.OP1 (on unit 21)

(note: only 1st 5 columns written here for the sake of clarity)

CG_LTM                  NCOLS =       18        NROWS =        6        FORM  =        2        PREC  =        2

1                       2                       3                       4                       5

1   -6.65821789802521E-05    1.29562159612018E-17   -6.47810798060089E-18   -1.29549999999999E-03    6.47766872193621E-05 .......

2   -2.99785601343913E-05   -1.96135553418977E-04    1.04193052213477E-04    1.39356777670951E-03   -6.70858061739371E-05 .......

3   -4.35697030582909E-05   -2.59100000000000E-03    1.30775055100798E-03    1.29550000000001E-03    6.19839872966866E-04 .......

4   -3.33844454038618E-04   -2.00000000000000E-02    9.80743672854175E-03    1.00000000000000E-02   -5.07064059129018E-03 .......

5    8.13687816036514E-03    1.47885176327023E-16   -7.39425881635114E-17   -7.78457159844592E-17   -5.93156091981744E-03 .......

6    5.63393757592496E-04    8.55130582230051E-17   -4.27565291115026E-17    9.99999999999996E-03    2.81696878796245E-04 .......

IF_LTM                  NCOLS =       18        NROWS =        8        FORM  =        2        PREC  =        2

1                       2                       3                       4                       5

1    6.02957424769077E-01    7.32039059471622E-02   -3.66019529735811E-02    3.35492666170908E-02   -7.19015457719424E-02 .......

2    7.32039059471623E-02    4.25469107253153E+00   -2.12163357113457E+00   -2.21879607113459E+00   -1.10665832128050E-01 .......

3   -3.66019529735811E-02   -2.12163357113457E+00    1.07224071582968E+00    1.10939803556729E+00    5.53329160640251E-02 .......

4    3.35492666170908E-02   -2.21879607113459E+00    1.10939803556729E+00    3.26418464157067E+00    1.75366508593570E-02 .......

5   -7.19015457719424E-02   -1.10665832128050E-01    5.53329160640251E-02    1.75366508593570E-02    4.96481812094837E-01 .......

6   -6.65046890695409E-01   -7.32039059471504E-02    3.66019529735752E-02   -1.24163383728600E+00    1.32307347677584E-01 .......

7   -1.34708893096271E-01   -2.21879607113459E+00    1.10939803556729E+00    2.54146535101691E-01    3.05700710026811E-02 .......

8    6.78737140960850E-02   -1.83869738075211E-01    9.19348690376054E-02    8.11498842422746E-02    2.62006997196796E-02 .......

MR                      NCOLS =        8        NROWS =        8        FORM  =        1        PREC  =        2

1                       2                       3                       4                       5

1    6.02957424769077E-01    7.32039059471622E-02   -3.66019529735811E-02    3.35492666170908E-02   -7.19015457719424E-02 .......

2    7.32039059471623E-02    4.25469107253153E+00   -2.12163357113457E+00   -2.21879607113459E+00   -1.10665832128050E-01 .......

3   -3.66019529735811E-02   -2.12163357113457E+00    1.07224071582968E+00    1.10939803556729E+00    5.53329160640251E-02 .......

4    3.35492666170908E-02   -2.21879607113459E+00    1.10939803556729E+00    3.26418464157067E+00    1.75366508593570E-02 .......

5   -7.19015457719424E-02   -1.10665832128050E-01    5.53329160640251E-02    1.75366508593570E-02    4.96481812094837E-01 .......

6   -6.65046890695409E-01   -7.32039059471504E-02    3.66019529735752E-02   -1.24163383728600E+00    1.32307347677584E-01 .......

7   -1.34708893096271E-01   -2.21879607113459E+00    1.10939803556729E+00    2.54146535101691E-01    3.05700710026811E-02 .......

8    6.78737140960850E-02   -1.83869738075211E-01    9.19348690376054E-02    8.11498842422746E-02    2.62006997196796E-02 .......

OUTPUT4 matrices requested in Exec Control to be written to file CB-EXAMPLE-12-b.OP2 (on unit 22)

(note: only 1st 5 columns written here the sake of clarity)

KRRGN                   NCOLS =       10        NROWS =       10        FORM  =        1        PREC  =        2

1                       2                       3                       4                       5

1    1.19504240447136E+03   -3.63797880709171E-12    1.81898940354586E-12    1.54614099301398E-11    5.97521202235677E+02 .......

2   -5.45696821063757E-12    0.00000000000000E+00    0.00000000000000E+00    1.81898940354586E-12    0.00000000000000E+00 .......

3    2.72848410531878E-12    0.00000000000000E+00    0.00000000000000E+00   -9.09494701772928E-13    0.00000000000000E+00 .......

4    2.08011385893769E-11    0.00000000000000E+00    0.00000000000000E+00   -1.16415321826935E-10    9.43778388773353E-12 .......

5    5.97521202235677E+02   -1.13686837721616E-13    5.68434188608080E-14   -1.59161572810262E-12    2.98760601117838E+02 .......

6   -1.19504240447137E+03    0.00000000000000E+00    0.00000000000000E+00   -1.79397829924710E-10   -5.97521202235685E+02 .......

7   -2.98427949019242E-13    0.00000000000000E+00    0.00000000000000E+00   -4.31782609666698E-10   -2.76401124210679E-12 .......

8   -5.97521202235677E+02   -1.81898940354586E-12    9.09494701772928E-13    1.36424205265939E-12   -2.98760601117839E+02 .......

9    0.00000000000000E+00    0.00000000000000E+00    0.00000000000000E+00    0.00000000000000E+00    0.00000000000000E+00 .......

10    0.00000000000000E+00    0.00000000000000E+00    0.00000000000000E+00    0.00000000000000E+00    0.00000000000000E+00 .......

RBMCG                   NCOLS =        6        NROWS =        6        FORM  =        2        PREC  =        2

1                       2                       3                       4                       5

1    2.41616914133782E+00   -3.35287353436797E-14   -6.52256026967279E-15   -1.34114941374719E-13   -3.97903932025656E-13 .......

2   -3.30846461338297E-14    2.41616914133786E+00    2.33146835171283E-14    7.74491581978509E-13    2.89102075612391E-13 .......

3   -6.52256026967279E-15    2.27734497926235E-14    2.41616914133783E+00   -9.59232693276135E-14   -7.10542735760100E-14 .......

4   -1.35891298214119E-13    7.81374964731185E-13   -1.24344978758018E-13    4.56169135583651E+03   -3.86535248253495E-12 .......

5   -3.92130772297605E-13    2.88435941797616E-13   -6.75015598972095E-14   -4.09272615797818E-12    4.53313153018053E+03 .......

6    1.99662508748588E-12    4.26325641456060E-14   -3.62376795237651E-13   -1.36424205265939E-11    2.85598256559946E+01 .......

MRRGN                   NCOLS =       10        NROWS =       10        FORM  =        1        PREC  =        2

1                       2                       3                       4                       5

1    6.02957424769077E-01    7.32039059471622E-02   -3.66019529735811E-02    3.35492666170908E-02   -7.19015457719424E-02 .......

2    7.32039059471623E-02    4.25469107253153E+00   -2.12163357113457E+00   -2.21879607113459E+00   -1.10665832128050E-01 .......

3   -3.66019529735811E-02   -2.12163357113457E+00    1.07224071582968E+00    1.10939803556729E+00    5.53329160640251E-02 .......

4    3.35492666170908E-02   -2.21879607113459E+00    1.10939803556729E+00    3.26418464157067E+00    1.75366508593570E-02 .......

5   -7.19015457719424E-02   -1.10665832128050E-01    5.53329160640251E-02    1.75366508593570E-02    4.96481812094837E-01 .......

6   -6.65046890695409E-01   -7.32039059471504E-02    3.66019529735752E-02   -1.24163383728600E+00    1.32307347677584E-01 .......

7   -1.34708893096271E-01   -2.21879607113459E+00    1.10939803556729E+00    2.54146535101691E-01    3.05700710026811E-02 .......

8    6.78737140960850E-02   -1.83869738075211E-01    9.19348690376054E-02    8.11498842422746E-02    2.62006997196796E-02 .......

9    1.22757372107055E-01   -1.75835189695839E+00    8.79175948479194E-01    1.25908689725916E+00    6.53537005701318E-02 .......

10    6.06162990294928E-01    1.82952442095713E-01   -9.14762210478567E-02   -4.91054200271590E-01   -1.36691428775775E-01 .......

RBRCG                   NCOLS =        6        NROWS =        8        FORM  =        2        PREC  =        2

1                       2                       3                       4                       5

1    1.00000000000000E+00    0.00000000000000E+00    0.00000000000000E+00    0.00000000000000E+00    5.37849392786371E+01 .......

2    0.00000000000000E+00    1.00000000000000E+00    0.00000000000000E+00   -5.37849392786371E+01    0.00000000000000E+00 .......

3    0.00000000000000E+00    0.00000000000000E+00    1.00000000000000E+00   -5.00000000000000E+01    0.00000000000000E+00 .......

4    0.00000000000000E+00    1.00000000000000E+00    0.00000000000000E+00   -3.78493927863709E+00    0.00000000000000E+00 .......

5    0.00000000000000E+00    0.00000000000000E+00    1.00000000000000E+00   -5.00000000000000E+01   -5.00000000000000E+01 .......

6    1.00000000000000E+00    0.00000000000000E+00    0.00000000000000E+00    0.00000000000000E+00    3.78493927863709E+00 .......

7    0.00000000000000E+00    1.00000000000000E+00    0.00000000000000E+00   -3.78493927863709E+00    0.00000000000000E+00 .......

8    0.00000000000000E+00    0.00000000000000E+00    1.00000000000000E+00   -5.00000000000000E+01    5.00000000000000E+01 .......

### Displ and Element force/stress OTM’s written to CB-EXAMPLE-12-b.OP8 and OP9

(OTM’s requested in Case Control)

CB-EXAMPLE-12-b.OP8 binary file of element force/stress OTM’s requested in Case Control

(note: only 1st 5 columns written here the sake of clarity)

OTM_ELFE                NCOLS =       18        NROWS =       16        FORM  =        2        PREC  =        2

1                       2                       3                       4                       5

1    2.09187572390564E-01    3.64063384390388E+00   -1.82031692195194E+00   -1.84227921264778E+00   -9.14925412689932E-01 .......

2    7.89453912890167E-01   -2.87503976462738E+00    1.43751988231369E+00    1.92080844772306E+00   -1.26234542491864E-01 .......

3    1.51560714339846E+00   -7.75207867487571E+00    3.87603933743785E+00    3.62690741509324E+00    1.45527637571713E+00 .......

4   -1.43934432738336E+00    4.48652751792572E+00   -2.24326375896286E+00   -2.73874759882899E+00    2.35906653084923E-01 .......

5   -1.84755627546901E-02    1.61117285562758E-01   -8.05586427813792E-02   -7.73459790410093E-02   -3.35197151472623E-02 .......

6    3.15199669918811E-02   -1.04108282913086E-01    5.20541414565432E-02    6.58960735567147E-02   -5.12144990278700E-03 .......

7    6.26679968599842E-01    1.90643492900070E+00   -9.53217464500349E-01   -1.19040949990613E-01   -1.14791218537626E-01 .......

8    9.67284596743351E-03   -5.33393540270422E-03    2.66696770135211E-03   -5.34876839175438E-02    8.35971431688627E-04 .......

9   -1.13315069892136E-01    3.78970456518829E+00   -1.89485228259414E+00   -1.26147862482940E+00   -9.55864075040792E-01 .......

10   -1.00896004659258E-02    2.99287680850590E+00   -1.49643840425295E+00   -4.03697533588189E+00   -1.41398274167766E-02 .......

11   -1.72540058669802E+00   -6.06107677196644E+00    3.03053838598322E+00    2.53928832803047E+00    1.96715396237338E+00 .......

12   -6.16614847670031E-02   -4.71348398353008E+00    2.35674199176504E+00    6.82365970711492E+00    3.39064169416761E-02 .......

13    2.27983320157212E-02    1.39311085669760E-01   -6.96555428348799E-02   -5.37509617215390E-02   -4.13377175157231E-02 .......

14    7.29336582157196E-04    1.08984399486375E-01   -5.44921997431877E-02   -1.53592573737906E-01   -6.79476503928156E-04 .......

15   -2.95361107284698E-01    1.80807707871691E+00   -9.04038539358453E-01   -1.95832712226347E+00    3.00896480121837E-03 .......

16   -4.72042770150405E-03    5.33393540270377E-03   -2.66696770135189E-03   -1.12160973347287E-01   -3.69369770142806E-03 .......

OTM_STRE                NCOLS =       18        NROWS =       18        FORM  =        2        PREC  =        2

1                       2                       3                       4                       5

1    0.00000000000000E+00    0.00000000000000E+00    0.00000000000000E+00    0.00000000000000E+00    0.00000000000000E+00 .......

2    0.00000000000000E+00    0.00000000000000E+00    0.00000000000000E+00    0.00000000000000E+00    0.00000000000000E+00 .......

3    0.00000000000000E+00    0.00000000000000E+00    0.00000000000000E+00    0.00000000000000E+00    0.00000000000000E+00 .......

4    0.00000000000000E+00    0.00000000000000E+00    0.00000000000000E+00    0.00000000000000E+00    0.00000000000000E+00 .......

5   -2.74867035744303E+00    7.58266712433821E+00   -3.79133356216910E+00   -1.07520478850513E+00    4.30045958649968E-01 .......

6   -2.74867035744303E+00    7.58266712433821E+00   -3.79133356216910E+00   -1.07520478850513E+00    4.30045958649968E-01 .......

7   -2.74867035744303E+00    7.58266712433821E+00   -3.79133356216910E+00   -1.07520478850513E+00    4.30045958649968E-01 .......

8   -1.00000000000000E+00   -1.00000000000000E+00   -1.00000000000000E+00   -1.00000000000000E+00   -1.00000000000000E+00 .......

9    0.00000000000000E+00    0.00000000000000E+00    0.00000000000000E+00    0.00000000000000E+00    0.00000000000000E+00 .......

10    0.00000000000000E+00    0.00000000000000E+00    0.00000000000000E+00    0.00000000000000E+00    0.00000000000000E+00 .......

11    0.00000000000000E+00    0.00000000000000E+00    0.00000000000000E+00    0.00000000000000E+00    0.00000000000000E+00 .......

12    0.00000000000000E+00    0.00000000000000E+00    0.00000000000000E+00    0.00000000000000E+00    0.00000000000000E+00 .......

13    0.00000000000000E+00    0.00000000000000E+00    0.00000000000000E+00    0.00000000000000E+00    0.00000000000000E+00 .......

14   -2.74867035744303E+00    7.58266712433821E+00   -3.79133356216910E+00   -1.07520478850513E+00    4.30045958649968E-01 .......

15   -2.74867035744303E+00    7.58266712433821E+00   -3.79133356216910E+00   -1.07520478850513E+00    4.30045958649968E-01 .......

16   -2.74867035744303E+00    7.58266712433821E+00   -3.79133356216910E+00   -1.07520478850513E+00    4.30045958649968E-01 .......

17   -1.00000000000000E+00   -1.00000000000000E+00   -1.00000000000000E+00   -1.00000000000000E+00   -1.00000000000000E+00 .......

18    1.00000000000000E+10    1.00000000000000E+10    1.00000000000000E+10    1.00000000000000E+10    1.00000000000000E+10 .......

CB-EXAMPLE-12-b.OT8 text file descriptor of rows in above binary file for element related OTM’s

This text file describes the rows of the elem related OTM matrices written to unformatted file: CB-EXAMPLE-12-b.OP8

----------------------------------------------------------------------------------------------

The description for each of the matrices has the headers:

ROW        : row number in the individual OTM described

DESCRIPTION: what OTM is this

TYPE       : element type

EID        : element ID

Then, for the element nodal force OTM:

GRID       : grid number of the element that the OTM is for

COMP       : displacement component number (1,2,3 translations and 4,5,6 rotations)

and for element engineering force and element stress OTMs:

ITEM       : element force or stress item (axial force, torque, etc)

The number of rows for each OTM depends on the output requests, by the user, in Case Control

The number of cols for each OTM depends on the number of support DOFs (NDOFR) and the number of eigenvecors (NVEC)where:

NDOFR =        8

NVEC  =        2

This text file has descriptions for the following element related OTMs from CB-EXAMPLE-12-b.OP8

Element engr  force OTM (matrix OTM_ELFE) with 2*NDOFR + NVEC =       18 cols

Element stress      OTM (matrix OTM_STRE) with 2*NDOFR + NVEC =       18 cols

---------------------------------------------------------------------------------

Explanation of rows of       16 row by       18 col matrix OTM_ELFE

ROW            DESCRIPTION             TYPE     EID              ITEM

------- ------------------------------ -------- -------    --------------------

1 Element engineering force      BAR          211    M1a: Mom Plane1 EndA

2 Element engineering force      BAR          211    M1b: Mom Plane2 EndA

3 Element engineering force      BAR          211    M2a: Mom Plane1 EndB

4 Element engineering force      BAR          211    M2b: Mom Plane2 EndB

5 Element engineering force      BAR          211    V1 : Shear Plane1

6 Element engineering force      BAR          211    V2 : Shear Plane2

7 Element engineering force      BAR          211    FX : Axial force

8 Element engineering force      BAR          211    T  : Torque

9 Element engineering force      BAR          212    M1a: Mom Plane1 EndA

10 Element engineering force      BAR          212    M1b: Mom Plane2 EndA

11 Element engineering force      BAR          212    M2a: Mom Plane1 EndB

12 Element engineering force      BAR          212    M2b: Mom Plane2 EndB

13 Element engineering force      BAR          212    V1 : Shear Plane1

14 Element engineering force      BAR          212    V2 : Shear Plane2

15 Element engineering force      BAR          212    FX : Axial force

16 Element engineering force      BAR          212    T  : Torque

---------------------------------------------------------------------------------

Explanation of rows of       18 row by       18 col matrix OTM_STRE

ROW            DESCRIPTION             TYPE     EID              ITEM

------- ------------------------------ -------- -------    --------------------

1 Element stress                 BAR          201    SA1: Stress Pt1 EndA

2 Element stress                 BAR          201    SA2: Stress Pt2 EndA

3 Element stress                 BAR          201    SA3: Stress Pt3 EndA

4 Element stress                 BAR          201    SA4: Stress Pt4 EndA

5 Element stress                 BAR          201    Axial Stress

6 Element stress                 BAR          201    SA-Max

7 Element stress                 BAR          201    SA-Min

8 Element stress                 BAR          201    MS-Tension

9 Element stress                 BAR          201    Torsional Stress

10 Element stress                 BAR          201    SB1: Stress Pt1 EndB

11 Element stress                 BAR          201    SB2: Stress Pt2 EndB

12 Element stress                 BAR          201    SB3: Stress Pt3 EndB

13 Element stress                 BAR          201    SB4: Stress Pt4 EndB

14 Element stress                 BAR          201    Axial stress

15 Element stress                 BAR          201    SB-Max

16 Element stress                 BAR          201    SB-Min

17 Element stress                 BAR          201    MS-Compression

18 Element stress                 BAR          201    MS-Torsion

CB-EXAMPLE-12-b.OP9 binary file of displacement OTM’s requested in Case Control

(note: only 1st 5 columns written here the sake of clarity)

OTM_ACCE                NCOLS =       10        NROWS =        6        FORM  =        2        PREC  =        2

1                       2                       3                       4                       5

1    2.19985250269592E-02    0.00000000000000E+00    0.00000000000000E+00   -5.00000000000004E-01    1.09992625134795E-02 .......

2   -2.02833087802606E-02    0.00000000000000E+00    0.00000000000000E+00    5.00000000000004E-01   -1.01416543901302E-02 .......

3   -1.68157865913898E-02   -1.00000000000000E+00    5.00000000000000E-01    5.00000000000005E-01    2.41592106704306E-01 .......

4   -3.36315731827796E-04   -2.00000000000000E-02    1.00000000000000E-02    1.00000000000001E-02   -5.16815786591390E-03 .......

5    8.00614495648658E-03    0.00000000000000E+00    0.00000000000000E+00    0.00000000000000E+00   -5.99692752175671E-03 .......

6    5.25433423070610E-04    0.00000000000000E+00    0.00000000000000E+00    9.99999999999992E-03    2.62716711535305E-04 .......

OTM_DISP                NCOLS =       18        NROWS =       12        FORM  =        2        PREC  =        2

1                       2                       3                       4                       5

1   -1.41293911043985E-05   -7.60029025912968E-05    3.80014512956484E-05    1.29492635368416E-04    3.14571590643487E-06 .......

2    1.62214021120513E-05    8.24359519633505E-05   -4.12179759816752E-05   -1.30161832591346E-04   -3.52963231517632E-06 .......

3    8.24222187730972E-05    3.12878663301563E-04   -1.56439331650781E-04   -2.40634384994669E-04   -1.68993616070736E-05 .......

4    5.88370868696758E-07    1.92529119983460E-06   -9.62645599917302E-07   -2.07019101770705E-06    1.88916538580397E-07 .......

5   -1.66743323917105E-06    2.22005501168008E-06   -1.11002750584004E-06   -1.14971054599053E-06   -8.88454144573320E-08 .......

6    5.12515138397389E-07    1.29205343624621E-07   -6.46026718123106E-08   -1.07589130445167E-06   -9.61720937623318E-08 .......

7    1.05104109813473E-05   -5.99087762260462E-05    2.99543881130231E-05    6.53233961326989E-05   -1.57813540011406E-06 .......

8   -9.46594436701425E-06    6.30861677743807E-05   -3.15430838871904E-05   -6.55217977160166E-05    1.38681670255135E-06 .......

9   -3.18288681491121E-06    3.22417925611894E-04   -1.61208962805947E-04   -1.96081126486432E-04   -3.61627931263323E-05 .......

10   -1.08618067423320E-07    3.64336233382231E-06   -1.82168116691115E-06   -2.63986785628832E-06   -3.24126419085498E-08 .......

11   -9.45071958677177E-07    4.90427017653186E-07   -2.45213508826593E-07   -2.21449664764883E-07    1.36502293189118E-07 .......

12    2.10600905814006E-07    3.21861205426993E-08   -1.60930602713497E-08   -6.09852683088454E-07   -3.82285587596693E-08 .......

CB-EXAMPLE-12-b.OT9 text file descriptor of rows in above binary file for grid related OTM’s

This text file describes the rows of the grid related OTM matrices written to unformatted file: CB-EXAMPLE-12-b.OP9

----------------------------------------------------------------------------------------------

The description for each of the matrices has the headers:

ROW        : row number in the individual OTM described

DESCRIPTION: what OTM is this

GRID       : grid number for this row of the OTM

COMP       : displacement component number (1,2,3 translations and 4,5,6 rotations)

The number of rows for each OTM depends on the output requests, by the user, in Case Control

The number of cols for each OTM depends on the number of support DOFs (NDOFR) and the number of eigenvecors (NVEC)where:

NDOFR =        8

NVEC  =        2

This text file has descriptions for the following grid relatad OTMs from CB-EXAMPLE-12b.OP9

Acceleration OTM (matrix OTM_ACCE) with   NDOFR + NVEC =       10 cols

Displacement OTM (matrix OTM_DISP) with 2*NDOFR + NVEC =       18 cols

---------------------------------------------------------------------------------

Explanation of rows of        6 row by       10 col matrix OTM_ACCE

ROW            DESCRIPTION              GRID     COMP

------- ------------------------------  -------    ----

1 Acceleration                         32       1

2 Acceleration                         32       2

3 Acceleration                         32       3

4 Acceleration                         32       4

5 Acceleration                         32       5

6 Acceleration                         32       6

---------------------------------------------------------------------------------

Explanation of rows of       12 row by       18 col matrix OTM_DISP

ROW            DESCRIPTION              GRID     COMP

------- ------------------------------  -------    ----

1 Displacement                         22       1

2 Displacement                         22       2

3 Displacement                         22       3

4 Displacement                         22       4

5 Displacement                         22       5

6 Displacement                         22       6

7 Displacement                         32       1

8 Displacement                         32       2

9 Displacement                         32       3

10 Displacement                         32       4

11 Displacement                         32       5

12 Displacement                         32       6

# Derivation of RBE3 Element Constraint Equations

## Introduction

The RBE3 element is used for distributing applied loads and mass from a reference point to other points  in the finite element model.  The geometry and loads for a RBE3 are shown in Figure 1.  Point d in the figure is the RBE3 reference (or dependent) point and is the grid where loads will be applied by the user.  The RBE3 element will distribute these loads to other, independent, points i = 1,…,N, in the model, where N is the total number of independent grid points defined on the RBE3 Bulk Data entry.  The RBE3 is not intended to add stiffness to the model as does a RBE2 element.  As such, the RBE3 reference point should not be a grid that is attached to other elements in the model – it should be a stand alone grid only connected to other grids through the REB3 element definition.  The following describes the nomenclature used in this appendix in deriving the “constraint” equations used in MYSTRAN for the RBE3 element.

Superscripts denote the location of a quantity:

“d” refers to the reference (or dependent) grid on the RBE3

“i” refers to the independent grids, the locations where the loads on point d will be distributed

For the sake of simplicity and clarity, the following derivation of the RBE3 equations is done for conditions where the global coordinate systems of all grid points involved in the RBE3 are the same and are rectangular.  The code in the MYSTRAN program is written for general conditions where the global system of all points may be different and non-rectangular.

Point i (1 to N)  is a typical point to which loads will be transferred from the reference point d via the RBE3

Point d is the RBE3 reference point  shown with the loads applied. The loads will be transferred to the points i (typical point i shown above)

Fig 1: RBE3 geometry and loads

## Equations for translational force components

In this section 3 equations will be developed that relate the forces applied at the RBE3 reference point to those where the loads will be distributed (points i = 1,…,N).

The sum of the forces on the points i = 1,…,N must equal the forces on the reference point d.  Thus:

The moments at reference point due to the forces at the points i are:

Write the , etc, as:

where  is the weighting factor (the WTi on the RBE3 Bulk Data entry) for the ith force and:

Equations 3 and 4 are sufficient for equations 1.  Substitute equations 3 and 4 into 2 to get the following 3 equations:

Define:

Using equation 8, equations 5-7 become:

The work done by the forces and moments at the reference point, d, is :

where  are the displacements and rotations of the reference point in the x, y, z directions.  Similarly, the work done by the forces on the points I = 1,…,N is:

The , ec, are the displacements in the x, y and z directions at point I.  Substitute equation 3 into 12 and 9, 10 and 11 into 12 and equate the work done by the two systems of forces:

Rearrange:

Since the ,  and are independent and, in general, not zero, equation 14 requires that:

Equation 15 represents 3 constraint equations for the RBE3. However, there are only 3 equations and 6 unknowns.  This will be resolved in the next section where we develop 3 more equations based on the moments at the reference point.

## Equations for rotational moment components

In addition to the 3 equations developed in the last section there are also 3 equations that relate the moments applied at the RBE3 reference point to those where the loads will be distributed (points i = 1,…,N).

Figure 2 shows how the forces in the y-z plane relate to the RBE3 reference point moment about the x axis:

radius to point i from ref point d in the y-z plane

Figure 2: Relationship of moments and forces in the y-z plane

Using the components of the forces, the moments about the x axis of the forces at the i = 1,..,N points is:

As before, express the forces at the i points using the weighting factors, :

Note that if equation 17 were substituted into 16 it would be seen that 17 is a valid representation of the tangential force components.

The work done by  must equal that due to all of the , or:

where is the tangential component of displacement at independent grid i in the y-z plane.  Substitute equation 17 into 18:

or:

From Figure 2 it can be seen that:

Therefore:

Define:

Substitute equations 20 and 21 into 19

In reference to Figures 3 and 4, define:

Then, and  , by similar reasoning for  in equation 22 are:

and

Thus, for the rotations:

Equations 15 and 26 constitute 6 equations in the 6 unknown displacements and rotations at point a.  They are summarized in matrix notation below at the end of this appendix.

radius to point i from ref point d in the z-x plane

Figure 3: Relationship of moments and forces in the z-x plane

radius to point i from ref point a in the x-y plane

Figure 4: Relationship of moments and forces in the x-y plane

## Summary of equations for the RBE3

In general, the equations for one RBE3 can be represented in matrix notation as:

is the square, d x d, matrix of coefficients for the dependent (or reference) grid denoted as REFGRID in field 4 of the RBE3 Bulk Data entry.  It can have up to d = 6 dependent components (REFC in field 5).  For all 6 components,  and are:

is a rectangular, d x N, matrix of coefficients for the N independent grids on the RBE3

A typical sub-matrix in is of size d by 3 with and . For d = 6:

A RBE3 is processed by solving equation 27 for the dependent degrees of freedom, , in terms of the independent degrees of freedom, .

# Equations for the BUSH Element

The stiffness equations for the BUSH element can be expressed as:

where  is a 12x12 matrix and   are the 12 degree of freedom (6 at each of the 2 grids) displacements and node forces. For the sake of clarity, rather than showing the whole 12x12 stiffness matrix, express the above equation in grid partitioned form as:

If we denote (i=1,…6) as the 6 stiffness values from the PBUSH Bulk Data entry then the above partitions are:

An image of the full 12x12 matrix with the above partitions is shown below:

Note that the partitions  are symmetric

The element engineering forces can be derived using the figure below:

The engineering forces in the BUSH element are:

This can be put into a form which includes all nodal forces as:

The 6x12 transformation matrix in the above equation is used in the MYSTRAN code to transform the element nodal forces to element engineering forces

The engineering forces in the BUSH element are:

This can be put into a form which includes all nodal forces as:

The 6x transformation matrix in the above equation is used in the MYSTRAN code to transform the element nodal forces to element engineering forces

Craig, R.R. and Bampton, M.C.C. “Coupling of Substructures for Dynamic Analysis”, AIAA Journal, Vol. 6, No. 7, July 1968, pp 1313-1319

Hurty, W.C. “Dynamic Analysis of Structural Systems Using Component Modes”, AIAA Journal, Vol. 3, No. 4, April 1965, pp 678-685

This should be verified by the user by inspection of the forces of single point constraint in the output from the analysis

Matrix size given in rows x columns where R means the size of the R-set, L is the size of the L-set, A is the size of the A-set, G is the size of the G-set and N is the number of eigenvectors.  See section 3.6 for definition of the complete displacement set notation